## Network Delay — Numerical Questions

Q1: Transmission + Propagation Delay
Link rate = 2 Mbps, packet size = 4000 bits, distance = 600 km, propagation speed = 3×10⁸ m/s. Calculate transmission delay, propagation delay, and total delay.

Q2: Store-and-Forward (3 Links, 2 Routers)
Packet = 3000 bits, each link rate = 1 Mbps, each link distance = 500 km, speed = 2×10⁸ m/s, processing delay = 1ms per router. Calculate total end-to-end delay.

Q3: N Packets Pipeline
10 packets, each 1000 bits, link rate = 1 Mbps, propagation delay = 20ms, no routers. Calculate total time until all packets reach destination.

Q4: Bottleneck Throughput
3 links: R1 = 10 Mbps, R2 = 2 Mbps, R3 = 8 Mbps. File size = 50 MB. Calculate total transfer time.

Q5: Lunar Rover Style
Earth to Moon distance = 3.8×10⁸ m, speed = 3×10⁸ m/s, packet = 8000 bits, link rate = 1 Mbps, 2 routers with processing delay = 5ms each. Calculate total end-to-end delay.

Q6: HTTP Non-Persistent
RTT = 30ms, object size = 100 KB, link rate = 1 Mbps. Calculate total time to download one object using non-persistent HTTP.

Q7: Multiple Links Different Rates
Host A → Router1 → Router2 → Host B

- Link1: rate = 512 Kbps, distance = 200 km

- Link2: rate = 1 Mbps, distance = 400 km

- Link3: rate = 256 Kbps, distance = 100 km

- Packet = 2048 bits, speed = 2×10⁸ m/s, no processing delay

Calculate total end-to-end delay.

### Topic 1: RTT & Timeout Calculation

Q1: Initial EstRTT = 100ms, DevRTT = 5ms, SampleRTT = 120ms, α=0.125, β=0.25. Calculate new Timeout Interval.

Q2: EstRTT = 80ms, DevRTT = 4ms. Two samples arrive: first 90ms then 200ms. Calculate Timeout after each sample.

Q3: EstRTT = 103ms, DevRTT = 6ms, SampleRTT = 110ms. Calculate only new DevRTT and new EstRTT.

Q4: After two RTT samples of 50ms and 75ms, starting from EstRTT = 60ms, DevRTT = 3ms. Find final Timeout Interval.

### 🔹 Topic 2: Sequence & ACK Numbers

Q1: ISN = 200, Segment1 = 20 bytes, Segment2 = 40 bytes, Segment3 = 15 bytes. Write Seq# and ACK# for each segment.

Q2: ISN_client = 300, ISN_server = 500. Write Seq# and ACK# for all 3 steps of TCP 3-way handshake.

Q3: Sender sends 1000 byte segments, ISN = 0. What is Seq# of 5th segment?

Q4: Host A sends 500KB file, MSS = 1000 bytes, ISN = 0. How many segments total? What is Seq# of last segment?

### 🔹 Topic 3: TCP Congestion Control

Q1: cwnd starts at 1 MSS, ssthresh = 8 MSS. Write cwnd value round by round until ssthresh is reached.

Q2: cwnd = 16 MSS, triple duplicate ACK received. TCP Reno: what is new cwnd and ssthresh? Write cwnd for next 4 RTTs.

Q3: cwnd = 12 MSS, timeout occurs. TCP Reno: what is new cwnd and ssthresh? How many RTTs to reach ssthresh again?

Q4: rwnd = 17KB, MSS = 1KB, RTT = 10ms. Packet drop at cwnd = 16KB (triple dup ACK). How many RTTs in Slow Start phase? Total time?

Q5: ssthresh = 8 MSS, cwnd = 1 MSS. How many RTTs does it take for cwnd to reach 12 MSS using TCP Reno?

### 🔹 Topic 4: IP Fragmentation

Q1: 4000 byte datagram (20 byte header), MTU = 1500. For each fragment write: Length, Offset, More flag, ID.

Q2: 3500 byte datagram (20 byte header), MTU = 1000. How many fragments? Write Offset for each.

Q3: Original datagram = 5000 bytes (20 byte header), MTU = 1200. Calculate all fragments completely.

Q4: Fragment 2 has Offset = 185. How many bytes of data were in Fragment 1?

Q5: MTU = 576 bytes, datagram = 2000 bytes (20 byte header). Calculate number of fragments and each offset value.

### 🔹 Topic 5: GBN / Selective Repeat Window Size

Q1: k = 4 bits. Calculate maximum window size for GBN and SR separately.

Q2: SR window size needed = 8. What is the minimum number of bits required in sequence number field?

Q3: k = 3 bits, GBN is used. Sender sends packets 0–6. ACK 3 is lost. How many packets does sender retransmit?

Q4: k = 2 bits (seq numbers: 0,1,2,3). What is max window for GBN? For SR? Why can SR not use window size 3?

### 🔹 Topic 6: Subnetting & CIDR

Q1: IP address = 192.168.10.0/26. How many subnets? How many hosts per subnet? Write first and last usable IP.

Q2: Network = 172.16.0.0/16. Divide into 8 equal subnets. What is new subnet mask? Write all 8 network addresses.

Q3: IP = 10.0.0.0/8. You need 500 hosts per subnet. What subnet mask will you use? How many subnets possible?

Q4: Given IP 192.168.1.130 with mask 255.255.255.192. Find: Network address, Broadcast address, Host range, Number of usable hosts.

### 🔹 Topic 7: Dijkstra (Link State Routing)

Q1: Graph with nodes A,B,C,D. Costs: A-B=2, A-C=5, B-C=1, B-D=4, C-D=2. Run Dijkstra from A. Find shortest path to all nodes.

Q2: 5 nodes: u,v,w,x,y. Edge costs given. Run Dijkstra from u and build complete forwarding table.

Q3: After running Dijkstra, shortest path from A to D = 7. Two paths exist: A→B→D (cost 3+5=8) and A→C→D (cost 2+5=7). Which path does router choose and why?

## All Solutions — Complete Step by Step

### 🔹 Topic 1: RTT & Timeout

Q1: EstRTT=100, DevRTT=5, SampleRTT=120, α=0.125, β=0.25

Step 1: DevRTT  = (0.75)(5) + (0.25)|120-100| = 3.75 + 5.00 = 8.75ms

Step 2: EstRTT  = (0.875)(100) + (0.125)(120) = 87.5 + 15 = 102.5ms

Step 3: Timeout = 102.5 + 4(8.75) = 102.5 + 35 = 137.5ms ✅

Q2: EstRTT=80, DevRTT=4 → Sample1=90ms, Sample2=200ms

After Sample1 = 90ms:

DevRTT  = (0.75)(4) + (0.25)|90-80|   = 3 + 2.5      = 5.5ms

EstRTT  = (0.875)(80) + (0.125)(90)   = 70 + 11.25   = 81.25ms

Timeout = 81.25 + 4(5.5)              = 81.25 + 22   = 103.25ms ✅

After Sample2 = 200ms:

DevRTT  = (0.75)(5.5) + (0.25)|200-81.25| = 4.125 + 29.69 = 33.81ms

EstRTT  = (0.875)(81.25) + (0.125)(200)   = 71.09 + 25    = 96.09ms

Timeout = 96.09 + 4(33.81)                = 96.09 + 135.24 = 231.33ms ✅

Q3: EstRTT=103, DevRTT=6, SampleRTT=110

DevRTT = (0.75)(6) + (0.25)|110-103| = 4.5 + 1.75 = 6.25ms

EstRTT = (0.875)(103) + (0.125)(110) = 90.125 + 13.75 = 103.875ms ✅

Q4: EstRTT=60, DevRTT=3 → Sample1=50, Sample2=75

After Sample1 = 50ms:

DevRTT  = (0.75)(3) + (0.25)|50-60|  = 2.25 + 2.5  = 4.75ms

EstRTT  = (0.875)(60) + (0.125)(50)  = 52.5 + 6.25 = 58.75ms

Timeout = 58.75 + 4(4.75)            = 58.75 + 19  = 77.75ms

After Sample2 = 75ms:

DevRTT  = (0.75)(4.75) + (0.25)|75-58.75| = 3.5625 + 4.0625 = 7.625ms

EstRTT  = (0.875)(58.75) + (0.125)(75)    = 51.41 + 9.375   = 60.78ms

Timeout = 60.78 + 4(7.625)                = 60.78 + 30.5    = 91.28ms ✅

### 🔹 Topic 2: Sequence & ACK Numbers

Q1: ISN=200, Seg1=20B, Seg2=40B, Seg3=15B

Segment 1: Seq = 200         → Receiver ACK = 220 (200+20)

Segment 2: Seq = 220         → Receiver ACK = 260 (220+40)

Segment 3: Seq = 260         → Receiver ACK = 275 (260+15) ✅

Q2: ISN_client=300, ISN_server=500 — 3-way handshake

Step 1 Client→Server: SYN,     Seq=300, ACK= —

Step 2 Server→Client: SYN-ACK, Seq=500, ACK=301

Step 3 Client→Server: ACK,     Seq=301, ACK=501 ✅

Q3: ISN=0, MSS=1000B → 5th segment Seq#?

Seg1: Seq=0, Seg2: Seq=1000, Seg3: Seq=2000

Seg4: Seq=3000, Seg5: Seq=4000 ✅

Q4: 500KB file, MSS=1000B, ISN=0

Total bytes    = 500 × 1024 = 512,000 bytes

Total segments = 512,000 / 1000 = 512 segments

Last Seq#      = (512-1) × 1000 = 511,000 ✅

### 🔹 Topic 3: TCP Congestion Control

Q1: cwnd=1MSS, ssthresh=8MSS — round by round

RTT1: cwnd = 1  (Slow Start)

RTT2: cwnd = 2  (Slow Start)

RTT3: cwnd = 4  (Slow Start)

RTT4: cwnd = 8  → ssthresh reached, switch to CA ✅

RTT5: cwnd = 9  (Congestion Avoidance: +1/RTT)

Q2: cwnd=16MSS, triple dup ACK — TCP Reno

After 3 dup ACK:

ssthresh = 16/2 = 8 MSS

cwnd     = 16/2 = 8 MSS  (Fast Recovery → CA)

Next 4 RTTs (CA: +1 per RTT):

RTT1: cwnd = 9

RTT2: cwnd = 10

RTT3: cwnd = 11

RTT4: cwnd = 12 ✅

Q3: cwnd=12MSS, timeout — TCP Reno

ssthresh = 12/2 = 6 MSS

cwnd     = 1 MSS (restart SS)

SS phase (cwnd doubles):

RTT1: cwnd = 1

RTT2: cwnd = 2

RTT3: cwnd = 4

RTT4: cwnd = 6 → ssthresh reached ✅

(Takes 4 RTTs to reach ssthresh again)

Q4: rwnd=17KB, MSS=1KB, RTT=10ms, drop at cwnd=16KB

SS phase (1→2→4→8→16):

RTT1:cwnd=1, RTT2:cwnd=2, RTT3:cwnd=4, RTT4:cwnd=8, RTT5:cwnd=16 → DROP

SS phase = 5 RTTs = 50ms

After drop (TCP Reno):

ssthresh = 8KB, cwnd = 8KB (CA phase starts)

To reach rwnd=17KB from cwnd=8KB:

17 - 8 = 9 more RTTs = 90ms ✅

Q5: ssthresh=8, cwnd=1 → how many RTTs to reach cwnd=12?

SS phase:

RTT1:cwnd=1, RTT2:cwnd=2, RTT3:cwnd=4, RTT4:cwnd=8 → CA starts

CA phase:

RTT5:cwnd=9, RTT6:cwnd=10, RTT7:cwnd=11, RTT8:cwnd=12 ✅

Total = 8 RTTs

### 🔹 Topic 4: IP Fragmentation

Q1: 4000B datagram (20B header), MTU=1500

Data = 4000 - 20 = 3980 bytes

Data/fragment = 1500 - 20 = 1480 bytes (1480/8=185 ✓)

Frag1: Length=1500, Offset=0,   More=1, ID=X, Data=1480B

Frag2: Length=1500, Offset=185, More=1, ID=X, Data=1480B

Frag3: Length=1040, Offset=370, More=0, ID=X, Data=1020B

Check: 1480+1480+1020 = 3980 ✅

Q2: 3500B datagram (20B header), MTU=1000

Data = 3500 - 20 = 3480 bytes

Max data/fragment = 1000-20 = 980B → not divisible by 8

Round down: 976B (976/8=122 ✓)

Fragments = ceil(3480/976) = 4 fragments

Frag1: Length=996,  Offset=0,   More=1, Data=976B

Frag2: Length=996,  Offset=122, More=1, Data=976B

Frag3: Length=996,  Offset=244, More=1, Data=976B

Frag4: Length=572,  Offset=366, More=0, Data=552B

Check: 976+976+976+552 = 3480 ✅

Q3: 5000B datagram (20B header), MTU=1200

Data = 5000 - 20 = 4980 bytes

Max data/fragment = 1200-20 = 1180B → not divisible by 8

Round down: 1176B (1176/8=147 ✓)

Fragments = ceil(4980/1176) = 5 fragments

Frag1: Length=1196, Offset=0,   More=1, Data=1176B

Frag2: Length=1196, Offset=147, More=1, Data=1176B

Frag3: Length=1196, Offset=294, More=1, Data=1176B

Frag4: Length=1196, Offset=441, More=1, Data=1176B

Frag5: Length=296,  Offset=588, More=0, Data=276B

Check: 1176×4+276 = 4980 ✅

Q4: Fragment 2 Offset=185 → how many bytes in Frag1?

Offset=185 means Frag2 starts at byte: 185 × 8 = 1480

Therefore Frag1 contained 1480 bytes of data ✅

Q5: MTU=576, datagram=2000B (20B header)

Data = 2000 - 20 = 1980 bytes

Max data/fragment = 576-20 = 556B → not divisible by 8

Round down: 552B (552/8=69 ✓)

Fragments = ceil(1980/552) = 4 fragments

Frag1: Length=572, Offset=0,   More=1, Data=552B

Frag2: Length=572, Offset=69,  More=1, Data=552B

Frag3: Length=572, Offset=138, More=1, Data=552B

Frag4: Length=344, Offset=207, More=0, Data=324B

Check: 552+552+552+324 = 1980 ✅

### 🔹 Topic 5: GBN / SR Window Size

Q1: k=4 bits

GBN max window = 2^4 - 1 = 15

SR  max window = 2^4 / 2  = 8 ✅

Q2: SR window=8, minimum bits needed?

N ≤ 2^k / 2

8 ≤ 2^k / 2  →  16 ≤ 2^k  →  k ≥ 4

Minimum k = 4 bits ✅

Q3: k=3 bits, GBN, packets 0-6 sent, ACK3 lost, timer expires

GBN window = 2^3 - 1 = 7

ACK3 lost → timer expires for packet 3

GBN retransmits from packet 3 onwards: 3, 4, 5, 6

Total retransmitted = 4 packets ✅

Q4: k=2 bits (seq: 0,1,2,3)

GBN max window = 2^2 - 1 = 3

SR  max window = 2^2 / 2  = 2

Why SR cannot use window=3:

Sender sends 0,1,2 → all ACKed, sender slides to send 3,0,1

Receiver window also slides to 3,0,1

If original ACKs were lost, receiver cannot tell if

new packet "0" is a retransmit or new one → AMBIGUITY ✅

### 🔹 Topic 6: Subnetting & CIDR

Q1: 192.168.10.0/26

Host bits   = 32-26 = 6

Subnets     = 2^(26-24) = 4  (from /24 parent)

Hosts/subnet = 2^6 - 2 = 62

First subnet 192.168.10.0/26:

Network:    192.168.10.0

First host: 192.168.10.1

Last host:  192.168.10.62

Broadcast:  192.168.10.63 ✅

Q2: 172.16.0.0/16, divide into 8 equal subnets

8 subnets = 2^3 → borrow 3 bits

New mask = /19 → 255.255.224.0

Subnet 1: 172.16.0.0/19

Subnet 2: 172.16.32.0/19

Subnet 3: 172.16.64.0/19

Subnet 4: 172.16.96.0/19

Subnet 5: 172.16.128.0/19

Subnet 6: 172.16.160.0/19

Subnet 7: 172.16.192.0/19

Subnet 8: 172.16.224.0/19 ✅

Q3: 10.0.0.0/8, need 500 hosts/subnet

2^h - 2 ≥ 500

h=9 → 2^9-2 = 510 hosts ✓ (h=8 gives only 254)

Subnet mask = 32-9 = /23 → 255.255.254.0

Number of subnets = 2^(23-8) = 2^15 = 32,768 ✅

Q4: IP=192.168.1.130, mask=255.255.255.192

/26 mask → block size = 64

130 falls in block 128 (128 to 191)

Network address:  192.168.1.128

Broadcast:        192.168.1.191

First host:       192.168.1.129

Last host:        192.168.1.190

Usable hosts:     2^6 - 2 = 62 ✅

### 🔹 Topic 7: Dijkstra (Link State)

Q1: A-B=2, A-C=5, B-C=1, B-D=4, C-D=2 → from A

Init:   D(A)=0, D(B)=∞, D(C)=∞, D(D)=∞

Step1 Visit A: D(B)=2, D(C)=5

Step2 Visit B: D(C)=min(5,2+1)=3, D(D)=min(∞,2+4)=6

Step3 Visit C: D(D)=min(6,3+2)=5

Step4 Visit D: Done

Results:

A→B = 2  (path: A→B)

A→C = 3  (path: A→B→C)

A→D = 5  (path: A→B→C→D) ✅

Q2: u-v=2, u-x=1, v-x=2, v-w=3, x-y=2, x-w=3, y-w=1 → from u

Init: D(u)=0, D(v)=∞, D(w)=∞, D(x)=∞, D(y)=∞

Step1 Visit u: D(v)=2, D(x)=1

Step2 Visit x: D(v)=min(2,3)=2, D(y)=3, D(w)=4

Step3 Visit v: D(w)=min(4,5)=4

Step4 Visit y: D(w)=min(4,4)=4

Step5 Visit w: Done

Forwarding Table from u:

Dest | Next Hop | Cost

v    | v        | 2

x    | x        | 1

y    | x        | 3

w    | x        | 4 ✅

Q3: Two paths A→D: via B=8, via C=7

Dijkstra picks MINIMUM cost path

Path A→C→D = cost 7 (chosen) ✅

Path A→B→D = cost 8 (rejected)

Router uses A→C→D because 7 < 8

### 🔹 Topic 8: Bellman-Ford (Distance Vector)

Q1: X-Y=4, Y-Z=1, X-Z=7

Initial Distance Vectors:

D_X: [X=0, Y=4, Z=7]

D_Y: [X=4, Y=0, Z=1]

D_Z: [X=7, Y=1, Z=0]

After Iteration 1:

D_X(Z) = min(7, D_X(Y)+D_Y(Z)) = min(7, 4+1) = 5 ← updated

D_Z(X) = min(7, D_Z(Y)+D_Y(X)) = min(7, 1+4) = 5 ← updated

Updated:

D_X: [X=0, Y=4, Z=5]

D_Y: [X=4, Y=0, Z=1]

D_Z: [X=5, Y=1, Z=0] ✅

Q2: A: D(B)=3, D(C)=7. B sends update D(C)=2

New D_A(C) via B = D_A(B) + D_B(C) = 3 + 2 = 5

Since 5 < 7 → A updates D(C) = 5 ✅

Q3: Y-X changes from 4 to 60. Count-to-infinity

Topology: Y-X=60(new), Y-Z=1, Z-X=50

Before change: D_Y(X)=4, D_Z(X)=5 (via Y)

Round 1: Y detects change

D_Y(X) = min(60, 1+D_Z(X)) = min(60,1+5) = 6 ← wrong! uses stale Z value

Round 2: Z learns from Y

D_Z(X) = min(50, 1+6) = 7

Round 3: Y updates

D_Y(X) = min(60, 1+7) = 8

This keeps going: 6,8,10,12...

D_Z: 7,9,11... until D_Z=50 (uses direct link)

Then D_Y = 51 → uses direct link (60) vs via Z(51) → picks 51

This slow increase = COUNT-TO-INFINITY problem ✅

Solution: Poisoned Reverse or Hold-down timers

### 🔹 Topic 9: Network Delay

Q1: L=4000bits, R=2Mbps, d=600km, s=3×10⁸

Tx delay   = L/R = 4000 / (2×10⁶) = 2ms

Prop delay = d/s = 600,000 / (3×10⁸) = 2ms

Total      = 2 + 2 = 4ms ✅

Q2: 3 links, 2 routers, L=3000bits, R=1Mbps, d=500km, s=2×10⁸, proc=1ms

Tx delay/link  = 3000/10⁶ = 3ms × 3 links = 9ms

Prop delay/link = 500,000/(2×10⁸) = 2.5ms × 3 = 7.5ms

Processing      = 2 × 1ms = 2ms

Total = 9 + 7.5 + 2 = 18.5ms ✅

Q3: 10 packets, 1000bits each, R=1Mbps, prop=20ms

Total = (N-1)×L/R + L/R + prop

= N×L/R + prop

= 10 × (1000/10⁶) + 20

= 10ms + 20ms = 30ms ✅

Q4: Bottleneck R=2Mbps, File=50MB

Transfer time = 50×8×10⁶ / (2×10⁶) = 400/2 = 200 seconds ✅

Q5: Lunar Rover — dist=3.8×10⁸m, s=3×10⁸, L=8000bits, R=1Mbps, 2 routers proc=5ms each

Tx delay   = 8000/10⁶ = 8ms

Prop delay = 3.8×10⁸ / 3×10⁸ = 1.267s = 1267ms

Processing = 2 × 5ms = 10ms

Total      = 8 + 1267 + 10 = 1285ms ≈ 1.285 sec ✅

Q6: Non-persistent HTTP, RTT=30ms, obj=100KB, R=1Mbps

Tx time = 100×1024×8 / 10⁶ = 819,200/1,000,000 = 0.8192s ≈ 820ms

Total   = 2×RTT + Tx = 60 + 820 = 880ms ✅

Q7: Multi-link, L=2048bits, s=2×10⁸, no proc delay

Tx1 = 2048/512,000  = 4ms    | Prop1 = 200,000/2×10⁸ = 1ms

Tx2 = 2048/1,000,000 = 2.05ms | Prop2 = 400,000/2×10⁸ = 2ms

Tx3 = 2048/256,000  = 8ms    | Prop3 = 100,000/2×10⁸ = 0.5ms

Total Tx   = 4 + 2.05 + 8 = 14.05ms

Total Prop = 1 + 2 + 0.5  = 3.5ms

Total      = 14.05 + 3.5  = 17.55ms ✅

Q8: a=100 pkt/sec, L=1000bits, R=1Mbps

Traffic Intensity = La/R = (1000×100) / 10⁶ = 0.1

La/R = 0.1 < 1 → Queuing delay is SMALL (manageable) ✅

Rule: La/R → 1 means queuing delay → infinity

### 🔹 Topic 10: Throughput

Q1: File=100MB, bottleneck=2Mbps

Transfer time = 100×8×10⁶ / 2×10⁶ = 800/2 = 400 seconds ✅

Q2: R1=10Mbps, R2=3Mbps, R3=8Mbps, 10 users

Bottleneck = min(10,3,8) = 3Mbps (shared among 10 users)

Per user   = 3/10 = 0.3 Mbps ✅

Q3: cwnd=8MSS, MSS=1460B, RTT=50ms

cwnd in bytes = 8 × 1460 = 11,680 bytes

Send rate = cwnd/RTT = 11,680 / 0.05 = 233,600 bytes/sec

= 233,600 × 8 bits = 1,868,800 bps ≈ 1.87 Mbps ✅