Question 1

MCQs, True/False, Fill in Blanks — Attempt on Answer Sheet  15 Marks — CLO-2

PART A: MCQs (circle the correct option)

1. The Sequence Number field in a TCP segment represents:
(a) the segment number being sent
(b) the byte-stream number of the first byte of data in that segment
(c) total number of bytes sent so far
(d) the ACK number from receiver

Answer: (b) — Seq# is byte-stream number of FIRST byte in segment. It is NOT segment number.

2. In TCP three-way handshake, which segment can carry data for the first time?
(a) SYN (Step 1)
(b) SYN-ACK (Step 2)
(c) ACK (Step 3)
(d) None of the above

Answer: (c) — Step 3 ACK may contain data. SYN and SYN-ACK carry no data.

3. The Receive Window field in TCP header is used for:
(a) Congestion control
(b) Error detection
(c) Sequencing of segments
(d) Flow control — receiver advertises how many bytes it can accept

Answer: (d) — Receive Window is for flow control.

4. TCP Header Length field is expressed in units of:
(a) bytes
(b) 16-bit words
(c) 32-bit words
(d) bits

Answer: (c) — Header Length is in 32-bit (4-byte) words. Typical value = 5 (20 bytes).

5. How many steps are in TCP connection teardown (connection closing)?
(a) 2 steps
(b) 3 steps
(c) 4 steps
(d) 1 step

Answer: (c) — 4 steps: Client FIN → Server ACK → Server FIN → Client ACK.


PART B: True / False

6. TCP Acknowledgement Number contains the byte-stream number of the last byte successfully received.

Answer: FALSE — ACK# = seq# of NEXT byte expected. Not last received byte.

7. TCP is a full-duplex protocol, meaning data flows bi-directionally on the same connection.

Answer: TRUE — Data flows in both directions on the same connection.

8. The TCP checksum covers only the TCP header, not the data payload.

Answer: FALSE — Checksum verifies integrity of the entire TCP segment (header + data).

9. In TCP, a shorter timeout value always leads to better performance.

Answer: FALSE — Too short timeout causes premature timeout and unnecessary retransmissions.

10. The SYN flag in TCP is used for connection setup.

Answer: TRUE — SYN Flag is used for connection setup.


PART C: Fill in the Blanks

11. In TCP, the minimum size of the fixed TCP header (without options) is _______ bytes.

Answer: 20 bytes

12. The TCP flag used for connection teardown is called _______ flag.

Answer: FIN flag

13. In TCP RTT estimation, the typical value of α used in the EstimatedRTT formula is _______.

Answer: 0.125

14. In a TCP connection, Seq# of first segment = 0, MSS = 1000 bytes. What is the Seq# of the 3rd segment?

Answer: 2000

15. The _______ option in TCP header is used so that sender and receiver negotiate segment size at connection setup.

Answer: MSS (Maximum Segment Size) Option

## Question 2

### Short Questions — Attempt any FIVE

## Q2.a

### List all the fields in the TCP header with their sizes (in bits) and the main purpose of each field. Draw the TCP header structure diagram.

[5 Marks]

### Answer:

TCP Header Fields:

| Field | Size | Purpose |
| --- | --- | --- |
| Source Port | 16 bits | Identifies sending application/process |
| Destination Port | 16 bits | Identifies receiving application/process |
| Sequence Number (Seq#) | 32 bits | Byte number of first data byte in segment |
| Acknowledgement Number (ACK#) | 32 bits | Indicates next byte expected from receiver |
| Header Length | 4 bits | Specifies TCP header size |
| Flags | 6 bits | Control bits (SYN, FIN, ACK, PSH, URG, RST) |
| Receive Window | 16 bits | Used for flow control |
| Checksum | 16 bits | Error detection of TCP segment |
| Urgent Pointer | 16 bits | Points to urgent data location |
| Options | Variable (up to 40 bytes) | Additional TCP options like MSS |

Header Size:

- Fixed TCP header = 20 bytes

- Maximum TCP header = 60 bytes (with options)

TCP Header Structure:

-------------------------------------------------
| Source Port        | Destination Port           |
 -------------------------------------------------
|              Sequence Number (32 bits)          |
 -------------------------------------------------
|        Acknowledgement Number (32 bits)          |
 -------------------------------------------------
|Header | Flags |       Receive Window             |
 -------------------------------------------------
| Checksum          |       Urgent Pointer         |
 -------------------------------------------------
|              Options (Variable)                  |
 -------------------------------------------------
|                    Data                          |
 -------------------------------------------------

# Q2.b

### Describe the TCP Three-Way Handshake with a neat timing diagram. What information is exchanged in each step?

[5 Marks]

### Answer:

TCP uses a three-way handshake to establish a connection.

### Step 1: Client → Server

Client                         Server

 SYN (Seq = x)  ------------>

- SYN flag = 1

- Client chooses Initial Sequence Number (ISN)

- No data is sent

### Step 2: Server → Client

Client                         Server

        <------------ SYN + ACK
        Seq = y, ACK = x+1

- SYN = 1 and ACK = 1

- Server sends its own ISN

- Acknowledges client's sequence number

- No data

### Step 3: Client → Server

Client                         Server

 ACK (ACK = y+1) ------------>

- ACK flag = 1

- Confirms server sequence number

- May contain data

Information exchanged:

- Sequence numbers

- MSS (Maximum Segment Size)

- Receive Window information

- Connection parameters

# Q2.c

### Explain the difference between TCP Sequence Numbers and Acknowledgement Numbers. Why does TCP use byte-stream numbers rather than segment numbers?

[5 Marks]

### Answer:

### Sequence Number:

- Represents the byte-stream number of the first byte of data in a TCP segment.

- It is not the segment number.

Example:

MSS = 1000 bytes

Segment 1 → Seq# = 0
Segment 2 → Seq# = 1000
Segment 3 → Seq# = 2000

### Acknowledgement Number:

- Shows the next byte expected from the sender.

- It is a cumulative acknowledgement.

Example:

If receiver gets bytes 0–999:

ACK = 1000

### Why byte-stream numbering?

TCP uses byte numbering because:

- Segments can have different sizes

- Data can arrive out of order

- Supports partial delivery

- Works even if MSS changes

# Q2.d

### What is the purpose of the TCP Receive Window field? How does it relate to flow control?

[5 Marks]

### Answer:

The Receive Window (rwnd) tells the sender how much data the receiver can accept.

- Receiver advertises rwnd value in ACK messages.

- Sender cannot send more than the available receive window.

Example:

Receive Window = 5000 bytes

Sender can have maximum 5000 bytes of unacknowledged data.

Purpose:

- Prevents sender from overwhelming receiver buffer.

- Provides TCP flow control.

# Q2.e

### Describe how TCP closes a connection. Draw the four-step connection teardown process with a timing diagram.

[5 Marks]

### Answer:

TCP connection closing uses a four-step teardown process.

Client                         Server

 FIN  ----------------------->

      <----------------------- ACK

      <----------------------- FIN

 ACK  ----------------------->

### Steps:

Step 1:
Client sends FIN to server.

Step 2:
Server sends ACK confirming FIN.

Step 3:
Server sends FIN when it finishes sending data.

Step 4:
Client sends ACK and connection closes.

This process is called half-close because one side can close while the other side continues sending data.

# Q2.f

### What is SampleRTT and EstimatedRTT in TCP? Why is EstimatedRTT preferred over SampleRTT for setting timeout?

[5 Marks]

### Answer:

### SampleRTT:

- Actual measured time between sending a segment and receiving its ACK.

- Does not include retransmitted segments.

### EstimatedRTT:

- Average RTT calculated using multiple SampleRTT values.

- Uses EWMA (Exponential Weighted Moving Average).

Formula:

EstimatedRTT = (1 − α) × EstimatedRTT + α × SampleRTT

Where:

α = 0.125

### Why EstimatedRTT is preferred?

SampleRTT changes frequently, causing:

- Unnecessary timeouts

- Extra retransmissions

EstimatedRTT provides:

- Smoother value

- Better timeout calculation

- Improved TCP performance

Question No -3

Long Numerical — Packet Delays and Throughput

A satellite communication system has Host A transmitting a 2 MB file to Host B. The path has three links with two intermediate routers. Link data rates are: R1 = 500 Kbps, R2 = 2 Mbps, R3 = 1 Mbps. Link distances are: d1 = 5,000 km (to geostationary satellite), d2 = 200 km, d3 = 800 km. Propagation speed on all links = 3×10⁸ m/s. Each router incurs a processing delay of 3 ms and queuing delay of 12 ms. Take 1 MB = 10⁶ bytes.

(a) Compute the total transmission delay for one packet of size 1500 bytes.

(b) Compute the total propagation delay for one packet.

(c) Compute the total end-to-end delay for one packet. (Show all components)

(d) If the 2 MB file is sent as a stream of 1500-byte packets back-to-back, how long does it take to deliver the LAST packet to Host B?

Question 4

TCP Sequence & Acknowledgement Numbers

15 Marks — CLO-3

Host A establishes a TCP connection with Host B. The Initial Sequence Number (ISN) for Host A is 300, and the ISN for Host B is 500. After the three-way handshake is complete, Host A sends three data segments to Host B with the following data sizes:

Segment 1: 30 bytes Segment 2: 50 bytes Segment 3: 25 bytes

(a) Draw the three-way handshake timing diagram showing Seq# and ACK# values exchanged.

(b) For each of the three data segments sent by Host A, write down the Seq# and ACK# values in the segment header.

(c) For each ACK sent by Host B in response to Host A's segments, write down the ACK# value.

(d) If Host B also sends 40 bytes of data to Host A piggybacked on the ACK for Segment 2, what Seq# does Host B use?

Question 5

TCP RTT Estimation and Timeout Calculation

15 Marks — CLO-3

A TCP connection is established between two hosts. At a certain point in time, the Estimated RTT = 90 ms and DevRTT = 7 ms. Two consecutive SampleRTT measurements are obtained: the first is 115 ms and the second is 60 ms. Use α = 0.125 and β = 0.25.

(a) Compute the new DevRTT and new EstimatedRTT after the first sample (115 ms). Then compute the Timeout Interval.

(b) Compute the new DevRTT and new EstimatedRTT after the second sample (60 ms). Then compute the new Timeout Interval.

(c) Compare the two Timeout Interval values. Briefly explain why the timeout increased or decreased after the second sample.

Question 6

TCP Header Analysis + Overview Features

15 Marks — CLO-2 & CLO-3

Answer the following parts regarding TCP operation:

(a) [5 marks]

A TCP segment is captured with the following header values: Source Port = 0x0050, Dest Port = 0x1F90, Seq# = 0x00000001, ACK# = 0x000003E8, Data Offset = 0x5, Flags = 0x018.

Determine:

Source and Destination port numbers in decimal

(ii) What do the flag values 0x018 indicate?

(iii) What is the header length in bytes?

(b) [5 marks]

Host A sends a 600 KB file to Host B over TCP. MSS = 1200 bytes. ISN = 0.

  - How many segments will TCP create?

(ii) What is the Seq# of the last segment?

(iii) If all segments are received and the last ACK is lost, what does the receiver do?

(c) [5 marks]

Explain the following TCP features:

  - Pipelined protocol — why is it used?

  - Byte stream oriented protocol — what does "no message boundaries" mean?

(iii) MSS Option — when and how is it negotiated?

## 📋 Week 11 — TCP Reliable Transfer, Flow Control & Congestion Control

### Predicted Final Term Paper — Dr. Nauman Mazhar, UCP

### ⬛ Question 1 — MCQs, T/F, Fill in Blanks (15 Marks | CLO-2)

#### Part A — MCQs (Circle correct option)

1. TCP retransmission is triggered by which TWO events?

- (a) Duplicate ACK and SYN timeout

- (b) Timeout and Triple Duplicate ACKs ✅

- (c) FIN flag and checksum error

- (d) Timeout and flow control violation

Answer: (b) — Slide 3: "Retransmissions triggered by: timeout events, duplicate ACK"

2. In TCP Fast Retransmit, the sender retransmits a segment after receiving:

- (a) 1 duplicate ACK

- (b) 2 duplicate ACKs

- (c) 3 duplicate ACKs ✅

- (d) Any out-of-order ACK

Answer: (c) — Slide 7: "if sender receives 3 ACKs for same data → resend unACKed segment"

3. In TCP Slow Start, how does cwnd grow every RTT?

- (a) Linearly — adds 1 MSS per RTT

- (b) Exponentially — doubles every RTT ✅

- (c) Stays constant until loss

- (d) Adds 2 MSS per RTT

Answer: (b) — Slide 20: "cwnd doubled every RTT"

4. TCP Reno and TCP Tahoe behave differently when:

- (a) Timeout occurs

- (b) SYN is received

- (c) Triple Duplicate ACKs occur ✅

- (d) FIN is received

Answer: (c) — Slide 24: Tahoe always sets cwnd=1 MSS. Reno cuts cwnd in half on 3-dup ACK.

5. The formula for TCP average throughput is:

- (a) W / RTT

- (b) W / 2RTT

- (c) 0.75 × W / RTT ✅

- (d) 2W / RTT

Answer: (c) — Slide 27: "Average throughput = 0.75 W/RTT"

#### Part B — True / False

6. TCP uses a separate retransmission timer for each unACKed segment. (T/F)

FALSE — Slide 3: "TCP uses a single retransmission timer, even if multiple unACKed segments exist"

7. In TCP Congestion Avoidance, cwnd increases by 1 MSS every RTT. (T/F)

TRUE — Slide 22: "cwnd increases by 1 MSS per RTT"

8. UDP is fair because it uses flow control to share bandwidth equally. (T/F)

FALSE — Slide 29: "UDP is not fair — send rate not affected by flow or congestion control"

9. TCP Fast Retransmit does NOT wait for timeout — it retransmits immediately after 3 dup ACKs. (T/F)

TRUE — Slide 7: "no further waiting for timeout"

10. When timeout occurs in TCP Reno, cwnd is cut to half. (T/F)

FALSE — Slide 24: On timeout cwnd = 1 MSS (not half). Half only happens on 3-dup ACK.

#### Part C — Fill in the Blanks

11. TCP receive window formula is: RcvWindow = RcvBuffer − [ _______ − _______ ]

Answer: LastByteRcvd − LastByteRead — Slide 13

12. When congestion is detected by 3-dup ACK in TCP Reno: ssthreshold = _______ and cwnd = _______

Answer: ssthreshold = cwnd/2, cwnd = cwnd/2 — Slide 25

13. The TCP approach to congestion control is _______ — end-systems infer congestion from loss/delay without router feedback.

Answer: End-to-end congestion control — Slide 17

14. AIMD stands for _______ Increase _______ Decrease.

Answer: Additive Increase, Multiplicative Decrease — Slide 26

15. A TCP sender limits its send rate using two variables: _______ and _______

Answer: cwnd (congestion window) and rwnd (receive window) — Slide 18

### ⬛ Question 2 — Short Questions (5×5 = 25 Marks | CLO-2)

#### Q2.a

Describe the TCP Receiver Events and ACK generation rules. Make a table showing all 4 events and the corresponding receiver action.

Key Points:

| Event at Receiver | Action |
| --- | --- |
| In-order segment, all prev ACKed | Delayed ACK — wait up to 500ms |
| In-order segment, one other pending | Immediately send single cumulative ACK |
| Out-of-order segment (gap detected) | Immediately send duplicate ACK, buffer segment |
| Segment fills gap from lower end | Immediately send ACK for largest in-order byte |

(Slide 6 — this exact table will appear in exam)

#### Q2.b

Explain TCP Flow Control. Write the RcvWindow formula and explain how it prevents receiver buffer overflow.

Key Points:

- Receiver has a fixed-size receive buffer

- App may read data slowly → buffer fills up

- Receiver advertises rwnd (spare room) in TCP header of every ACK

- Formula: RcvWindow = RcvBuffer − [LastByteRcvd − LastByteRead]

- Sender rule: LastByteSent − LastByteAcked ≤ rwnd

- This guarantees receive buffer will never overflow (Slides 12–15)

#### Q2.c

What is the difference between TCP Slow Start and Congestion Avoidance? When does TCP switch from SS to CA?

Key Points:

|  | Slow Start | Congestion Avoidance |
| --- | --- | --- |
| When | Connection start OR after timeout | When cwnd ≥ ssthreshold |
| Growth | Exponential (double per RTT) | Linear (+1 MSS per RTT) |
| Why | Fast ramp-up from idle | Probe carefully near congestion |

- Switch: when cwnd reaches ssthreshold (Slides 20–22)

#### Q2.d

Explain TCP Fast Retransmit. Why is it better than waiting for a timeout?

Key Points:

- Timeout period is long → causes poor performance if we wait

- 3 Duplicate ACKs = implicit NAK for missing segment

- Sender immediately retransmits unACKed segment with smallest seq#

- No need to wait for timer to expire

- Why 3? — 1 or 2 dup ACKs can occur naturally from reordering; 3 is a strong signal of loss (Slide 7)

#### Q2.e

Explain the difference between TCP Reno and TCP Tahoe when a loss event occurs.

Key Points:

| Event | TCP Tahoe | TCP Reno |
| --- | --- | --- |
| Timeout | cwnd = 1 MSS, SS from start | cwnd = 1 MSS, SS from start |
| 3 Dup ACKs | cwnd = 1 MSS, SS from start | cwnd = cwnd/2, Fast Recovery (CA) |

- Reno treats 3-dup ACK less severely because network can still deliver some segments

- Tahoe treats both events identically (more conservative) (Slide 24)

#### Q2.f

What is AIMD? Explain how it helps TCP achieve fair bandwidth sharing.

Key Points:

- Additive Increase: cwnd += 1 MSS per RTT when no loss → probes for available bandwidth

- Multiplicative Decrease: cwnd = cwnd/2 when loss detected → backs off quickly

- AIMD sawtooth pattern: keeps probing and backing off

- Result: K TCP sessions on same bottleneck link of bandwidth R → each gets R/K (Slides 26–28)

### ⬛ Question 3 — Delays Numerical (15 Marks | CLO-2)

A file of 3 MB is to be sent from Host A to Host B over a path with 3 links and 2 routers. Link data rates: R1 = 1 Mbps, R2 = 5 Mbps, R3 = 2 Mbps. Link distances: d1 = 2000 km, d2 = 500 km, d3 = 1000 km. Propagation speed = 2.5×10⁸ m/s. Each router has processing delay = 4 ms and queuing delay = 8 ms. Packet size = 1500 bytes. Take 1 MB = 10⁶ bytes.

(a) Compute total transmission delay for one packet.
(b) Compute total propagation delay for one packet.
(c) Compute total end-to-end delay for one packet.
(d) How long to deliver ALL packets of the 3 MB file?

Complete Solution:

L = 1500 × 8 = 12,000 bits

(a) Transmission Delays:

Tx1 = 12,000 / 1,000,000      = 12 ms

Tx2 = 12,000 / 5,000,000      = 2.4 ms

Tx3 = 12,000 / 2,000,000      = 6 ms

Total Tx = 12 + 2.4 + 6       = 20.4 ms

(b) Propagation Delays:

Prop1 = 2,000,000 / 2.5×10⁸  = 8 ms

Prop2 = 500,000 / 2.5×10⁸    = 2 ms

Prop3 = 1,000,000 / 2.5×10⁸  = 4 ms

Total Prop = 8 + 2 + 4        = 14 ms

(c) Router delays = 2 × (4 + 8) = 24 ms

Total 1 pkt = 20.4 + 14 + 24  = 58.4 ms

(d) N = (3×10⁶ × 8) / 12,000  = 2000 packets

Tx at source for (N−1) pkts = 1999 × 12 ms = 23,988 ms

Total = 23,988 + 58.4         = 24,046.4 ms ≈ 24.05 sec

### ⬛ Question 4 — TCP Congestion Control Graph Analysis (15 Marks | CLO-3)

A TCP Reno connection shows the following congestion window (cwnd) behavior across rounds:

Round:  1   2   3   4   5   6   7   8   9   10  11  12  13  14

cwnd:   1   2   4   8   6   7   8   9  10   11   6   7   8   9

ssthreshold starts at 16 MSS.

(a) Identify all intervals where Slow Start is operating.
(b) Identify all intervals where Congestion Avoidance is operating.
(c) At which round does a loss event occur? What type — timeout or 3-dup ACK?
(d) What is the value of ssthreshold after the loss event?
(e) What is the value of cwnd immediately after the loss event (Round 5)?

Complete Solution:

(a) Slow Start: Rounds 1–4

cwnd = 1,2,4,8 → doubling = exponential = SS

(b) Congestion Avoidance: Rounds 5–10 and Rounds 11–14

cwnd = 6,7,8,9,10,11 → +1 per round = linear = CA

(c) Loss event between Round 4 and Round 5

cwnd went from 8 → dropped, then resumed at 6 (not 1)

→ Since cwnd = cwnd/2 = 4? No, new cwnd = 6...

Actually: cwnd at loss = ~12 (implied ssthresh=6 means cwnd was 12 at loss)

Type: Triple Duplicate ACK (because cwnd halved, not reset to 1)

(d) ssthreshold after loss = cwnd/2 at time of loss

If cwnd was 12 at loss → ssthreshold = 6

(e) cwnd immediately after (TCP Reno, 3-dup ACK) = cwnd/2 = 6

(TCP Tahoe would give cwnd = 1)

### ⬛ Question 5 — TCP Slow Start Graph + Calculations (15 Marks | CLO-3)

A TCP Reno connection has MSS = 1 KB, RTT = 12 ms, rwnd = 20 KB. The connection starts with Slow Start. A packet drop (triple duplicate ACK) occurs when cwnd = 16 KB. After the drop, there are no further loss events.

(a) Draw the cwnd vs RTT graph showing SS phase, drop event, and CA phase until cwnd reaches rwnd.
(b) How many RTTs was the connection in Slow Start? What is that in milliseconds?
(c) After the packet drop, how many RTTs does it take for cwnd to reach rwnd (20 KB)? What is that in ms?
(d) What is the average throughput during the CA phase after the drop?

Complete Solution:

(a) Graph values:

RTT 1: cwnd = 1 KB   ← SS starts

RTT 2: cwnd = 2 KB

RTT 3: cwnd = 4 KB

RTT 4: cwnd = 8 KB

RTT 5: cwnd = 16 KB → PACKET DROP HERE (3-dup ACK)

ssthreshold = 16/2 = 8 KB

cwnd = 16/2 = 8 KB  (TCP Reno)

RTT 6: cwnd = 9 KB   ← CA starts (+1 per RTT)

RTT 7: cwnd = 10 KB

RTT 8: cwnd = 11 KB

RTT 9: cwnd = 12 KB

RTT 10: cwnd = 13 KB

RTT 11: cwnd = 14 KB

RTT 12: cwnd = 15 KB

RTT 13: cwnd = 16 KB

RTT 14: cwnd = 17 KB

RTT 15: cwnd = 18 KB

RTT 16: cwnd = 19 KB

RTT 17: cwnd = 20 KB = rwnd → DONE

(b) SS phase = RTTs 1–5 = 5 RTTs

In ms = 5 × 12 = 60 ms

(c) After drop: cwnd = 8 KB, need to reach 20 KB

RTTs needed = 20 − 8 = 12 RTTs

In ms = 12 × 12 = 144 ms

(d) Average throughput during CA phase:

Data sent ≈ average cwnd × RTTs / total time

Average cwnd = (8+20)/2 = 14 KB

Throughput ≈ 14 KB / 12 ms = 14×8×1000 / 12 = 9,333 Kbps ≈ 9.3 Mbps

### ⬛ Question 6 — Flow Control + Fairness (15 Marks | CLO-3)

Answer the following parts:

(a) [5 marks] A TCP receiver has RcvBuffer = 8000 bytes. At a certain moment: LastByteRcvd = 6500, LastByteRead = 4000. Calculate the RcvWindow. What does the sender do with this value?

(b) [5 marks] Explain TCP Fairness. If 3 TCP connections share a bottleneck link of 12 Mbps, what rate does each get? Now if one host opens 4 parallel TCP connections, how does that affect fairness?

(c) [5 marks] A TCP sender knows: cwnd = 10 MSS, rwnd = 7 MSS, MSS = 1000 bytes. How many bytes can the sender have unACKed at any time? Why is this constraint important?

Complete Solutions:

(a) RcvWindow = RcvBuffer − [LastByteRcvd − LastByteRead]

= 8000 − [6500 − 4000]

= 8000 − 2500

= 5500 bytes

Sender will not send more than 5500 bytes of unACKed data.

Guarantees buffer does not overflow. (Slide 13–15)

(b) TCP Fairness: K sessions sharing bandwidth R → each gets R/K

3 connections × 12 Mbps → each gets 4 Mbps

If 1 host opens 4 parallel TCPs:

- That host = 4 out of 7 total TCP connections

- Gets 4/7 × 12 = 6.86 Mbps (unfair — more than others)

- Single-connection hosts get only 12/7 = 1.71 Mbps each

→ Parallel connections VIOLATE fairness (Slide 29)

(c) Sender can have unACKed data = min(cwnd, rwnd)

= min(10, 7) MSS = 7 MSS = 7000 bytes

Important because:

- cwnd controls congestion (network capacity)

- rwnd controls flow (receiver capacity)

- Both must be satisfied to protect both network AND receiver

### 📌 Quick Revision — Week 11 Ke Most Important Points

| Topic | Jo yaad rakhna ZAROOR hai |
| --- | --- |
| Slow Start | cwnd = 1 MSS, doubles every RTT, exponential |
| CA | cwnd + 1 MSS per RTT, linear |
| Timeout | cwnd = 1 MSS, ssthresh = cwnd/2, restart SS |
| 3-dup ACK (Reno) | cwnd = cwnd/2, ssthresh = cwnd/2, Fast Recovery (CA) |
| 3-dup ACK (Tahoe) | cwnd = 1 MSS (same as timeout) |
| Flow Control | rwnd advertised by receiver, sender ≤ rwnd unACKed |
| Throughput | 0.75 × W / RTT |
| AIMD | Additive increase + Multiplicative decrease |
| Fairness | K connections → R/K each (parallel TCPs break this) |

## 📋 Week 12 — Network Layer, Datagrams, Router & IP

### Predicted Final Term Paper — Dr. Nauman Mazhar, UCP

### ⬛ Question 1 — MCQs, T/F, Fill in Blanks (15 Marks | CLO-2)

#### Part A — MCQs (Circle correct option)

1. The main difference between Forwarding and Routing in the network layer is:

- (a) Forwarding builds routing tables; Routing moves packets

- (b) Forwarding moves packet from input to output port of ONE router; Routing determines end-to-end path ✅

- (c) Both are the same function

- (d) Routing is done at transport layer; Forwarding at network layer

Answer: (b) — Slide 6: Forwarding = "process of getting through a single interchange." Routing = "planning the trip from source to dest."

2. In a Datagram Network, packets are forwarded using:

- (a) Virtual circuit numbers

- (b) Port numbers

- (c) Destination IP address ✅

- (d) MAC addresses

Answer: (c) — Slide 10: "Packets forwarded using address of destination host"

3. In IP header, the Protocol field value 6 indicates the payload is:

- (a) UDP

- (b) ICMP

- (c) TCP ✅

- (d) DNS

Answer: (c) — Slide 24: "TCP=6, UDP=17"

4. IP fragmentation is performed by:

- (a) Source host only

- (b) Destination host only

- (c) Routers along the path ✅

- (d) Transport layer

Answer: (c) — Slide 28: "larger IP datagrams divided (fragmented) by routers"

5. IP datagram reassembly is performed by:

- (a) Every router along the path

- (b) The first router after fragmentation

- (c) Only at the final destination host ✅

- (d) The source host

Answer: (c) — Slide 28: "reassembled only at final destination"

6. The Fragment Offset field in IP header is measured in units of:

- (a) Bytes

- (b) Bits

- (c) 8 bytes (64 bits) ✅

- (d) 32-bit words

Answer: (c) — Slide 23: "measured in 64 bit units (8 bytes)"

7. When a router's TTL field reaches 0, the router:

- (a) Sends the packet to the next hop anyway

- (b) Discards the datagram ✅

- (c) Sends it back to source

- (d) Resets TTL to 64

Answer: (b) — Slide 24: "when TTL = 0, router discards datagram"

#### Part B — True / False

8. In a Virtual Circuit Network, packets are forwarded using the destination IP address. (T/F)

FALSE — Slide 9: VC networks use virtual circuit numbers. Datagram networks use dest IP addr.

9. IP Header Checksum covers both the IP header AND the data payload. (T/F)

FALSE — Slide 25: Checksum is applied only to IP header, not to data.

10. In Longest Prefix Matching, the router selects the forwarding table entry with the shortest matching prefix. (T/F)

FALSE — Slide 13: It uses the LONGEST prefix that matches.

11. The Don't Fragment bit in IP flags, if set, prohibits routers from fragmenting that datagram. (T/F)

TRUE — Slide 23: "Don't Fragment bit: prohibits fragmentation"

12. All fragments of a fragmented IP datagram carry the same Identification number as the original datagram. (T/F)

TRUE — Slide 31: "each resulting fragment is stamped with...identification number of original datagram"

#### Part C — Fill in the Blanks

13. The minimum size of an IP header (without options) is _______ bytes, and the Header Length field value in that case is _______.

Answer: 20 bytes, value = 5 (in 32-bit words) — Slide 21

14. The More flag bit is set to _______ in the last fragment of a fragmented IP datagram.

Answer: 0 (zero) — Slide 32

15. The maximum theoretical size of an IP datagram according to the Total Length field is _______ bytes.

Answer: 65,535 bytes — Slide 22

### ⬛ Question 2 — Short Questions (5×5 = 25 Marks | CLO-2)

#### Q2.a

Explain the two key functions of the Network Layer: Forwarding and Routing. Use the trip planning analogy from your slides.

Key Points:

- Forwarding: Moving a packet from input port to output port at a SINGLE router. Uses the forwarding table. Fast, done in hardware. Analogy: "getting through a single interchange"

- Routing: Determining the end-to-end path from source to destination. Runs routing algorithms (RIP, OSPF, BGP). Builds routing table → populates forwarding table. Analogy: "planning the trip from source to dest"

- Routing algo → Routing Table → Forwarding Table → used for Forwarding (Slide 6–7)

#### Q2.b

Compare Virtual Circuit Networks and Datagram Networks. Which one does the Internet use and why?

Key Points:

| Feature | Virtual Circuit Network | Datagram Network |
| --- | --- | --- |
| Service | Connection-oriented | Connectionless |
| Packet forwarding | Using VC numbers | Using dest IP address |
| Connection setup | Required (call setup) | NOT required |
| Router state | Maintained per connection | NOT maintained |
| Example | ATM, Frame Relay | Internet (IP) |

- Internet uses Datagram — no call setup, each packet treated independently, scalable (Slides 9–10)

#### Q2.c

Describe the IP Datagram format. List any SIX header fields with their sizes and purposes.

Key Points (pick any 6 from this list):

| Field | Size | Purpose |
| --- | --- | --- |
| Version | 4 bit | IPv4 or IPv6 |
| Header Length | 4 bit | Header size in 32-bit words; min=5 (20 bytes) |
| TOS | 8 bit | Packet priority / service type |
| Total Length | 16 bit | Header + data in bytes; max 65,535 |
| Identification | 16 bit | Unique ID for fragmentation |
| Flags | 3 bit | DF bit, More bit for fragmentation |
| Fragment Offset | 13 bit | Position in original datagram (in 8-byte units) |
| TTL | 8 bit | Max hops; decremented by each router |
| Protocol | 8 bit | TCP=6, UDP=17 |
| Header Checksum | 16 bit | Error detection on header only |
| Src/Dest IP | 32 bit each | Source and destination addresses |

(Slides 21–27)

#### Q2.d

What is IP Fragmentation? Why is it needed? Who performs fragmentation and who performs reassembly?

Key Points:

- Every link has an MTU (Maximum Transfer Unit) — largest IP packet it can carry

- Different links have different MTUs (e.g., Ethernet MTU = 1500 bytes)

- If a datagram is larger than MTU, it must be fragmented

- Routers perform fragmentation along the path

- Only the final destination host performs reassembly

- All fragments carry the same Identification number

- Last fragment has More bit = 0; all others have More bit = 1

- Offset field tells where each fragment belongs in original datagram (Slides 28–32)

#### Q2.e

Explain Longest Prefix Matching in IP forwarding tables. Why is this technique used?

Key Points:

- Router's forwarding table contains address ranges with prefixes

- When a packet arrives, router matches the destination IP against all prefixes

- It selects the entry with the LONGEST matching prefix (most specific match)

- Why: A single IP address can match multiple entries (e.g., a /24 and a /16 route). The longest prefix = most specific = best route.

- Example: Dest IP = 11001000 00010111 00011000 10101010

  - Matches /16 prefix AND /24 prefix → use /24 (longer) (Slides 12–13)

#### Q2.f

Describe the Router Architecture. What are the two key functions a router performs?

Key Points:

- Two key functions: (1) Run routing algorithms/protocols (RIP, OSPF, BGP) to build routing tables, (2) Process and forward datagrams from input to output

- Input Port: Receives bits → Link layer processing → Forwarding table lookup → send to switch fabric

- Switch Fabric: Connects input ports to output ports

- Output Port: Queuing (if arrival rate > output rate) → encapsulation → send bits

- Queuing occurs at output port when datagrams arrive faster than output data rate (Slides 16–18)

### ⬛ Question 3 — Delays Numerical (15 Marks | CLO-2)

Host A wants to send a 5 MB file to Host B. The path has 3 links and 2 routers. Data rates: R1 = 2 Mbps, R2 = 8 Mbps, R3 = 4 Mbps. Distances: d1 = 1000 km, d2 = 4000 km, d3 = 500 km. Propagation speed = 3×10⁸ m/s. Each router: processing delay = 2 ms, queuing delay = 10 ms. Packet size = 2000 bytes. 1 MB = 10⁶ bytes.

(a) Total transmission delay for one packet.
(b) Total propagation delay for one packet.
(c) Total end-to-end delay for one packet.
(d) Total time to deliver all packets of the 5 MB file.

Complete Solution:

L = 2000 × 8 = 16,000 bits

(a) Transmission Delays:

Tx1 = 16,000 / 2,000,000     = 8 ms

Tx2 = 16,000 / 8,000,000     = 2 ms

Tx3 = 16,000 / 4,000,000     = 4 ms

Total Tx = 8 + 2 + 4          = 14 ms

(b) Propagation Delays:

Prop1 = 1,000,000 / 3×10⁸   = 3.33 ms

Prop2 = 4,000,000 / 3×10⁸   = 13.33 ms

Prop3 = 500,000 / 3×10⁸     = 1.67 ms

Total Prop = 3.33+13.33+1.67  = 18.33 ms

(c) Router delays = 2 × (2+10) = 24 ms

Total (1 pkt) = 14+18.33+24   = 56.33 ms

(d) N = (5×10⁶ × 8) / 16,000  = 2500 packets

Tx at source for (N−1) pkts

= 2499 × 8 ms                 = 19,992 ms

Total = 19,992 + 56.33        = 20,048.33 ms

≈ 20.05 seconds

### ⬛ Question 4 — IP Fragmentation Calculation (15 Marks | CLO-2)

A router receives an IP datagram of total size 5000 bytes (including 20 bytes IP header). The outgoing link has an MTU of 1500 bytes. The datagram has Identification number = 422.

(a) How many fragments will be created?
(b) For each fragment, determine: Total Length, Identification, Fragment Offset, and More flag value.
(c) Which fragment carries the original datagram's Identification number?
(d) At which point are these fragments reassembled?

Complete Solution:

Original data payload = 5000 − 20 = 4980 bytes

Each fragment header = 20 bytes

Max data per fragment = 1500 − 20 = 1480 bytes

(1480 must be multiple of 8 → 1480/8 = 185 ✓)

Number of fragments = ceil(4980 / 1480) = 4 fragments

Fragment 1: 1480 bytes data

Fragment 2: 1480 bytes data

Fragment 3: 1480 bytes data

Fragment 4: 4980 − (3×1480) = 540 bytes data

Fragment table:

| Frag | Total Length    | ID  | Offset         | More |

|------|-----------------|-----|----------------|------|

|  1   | 1480+20 = 1500  | 422 | 0              |  1   |

|  2   | 1480+20 = 1500  | 422 | 1480/8 = 185   |  1   |

|  3   | 1480+20 = 1500  | 422 | 2960/8 = 370   |  1   |

|  4   | 540+20  = 560   | 422 | 4440/8 = 555   |  0   |

Verify: 1480+1480+1480+540 = 4980 ✓

(c) ALL fragments carry ID = 422 (same as original)

(d) Reassembly happens ONLY at the final destination host,

NOT at intermediate routers.

### ⬛ Question 5 — IP Header Fields Analysis (15 Marks | CLO-2)

A captured IP datagram has the following header field values (in hex): Version=4, IHL=5, TOS=00, Total Length=0028, ID=1A2B, Flags=010 (binary), Offset=0, TTL=40, Protocol=11, Header Checksum=XXXX, Src IP=C0A80001, Dest IP=C0A800FE.

(a) What IP version is this? What is the header length in bytes?
(b) What is the total datagram length in bytes?
(c) What does the Flags value 010 mean? Can this datagram be fragmented?
(d) What transport layer protocol is carrying the data? What is the TTL value in decimal?
(e) Convert Src IP C0A80001 to dotted decimal notation.

Complete Solution:

(a) Version = 4 → IPv4

IHL = 5 → Header length = 5 × 4 = 20 bytes (no options)

(b) Total Length = 0x0028 = 40 bytes

(c) Flags = 010 binary → bit1=1 = Don't Fragment bit SET

This datagram CANNOT be fragmented by any router.

If it exceeds an MTU, router must DROP it and send ICMP error.

(d) Protocol = 0x11 = 17 decimal → UDP

TTL = 0x40 = 64 decimal (default TTL value)

(e) C0A80001 → split into bytes:

C0 = 192, A8 = 168, 00 = 0, 01 = 1

Src IP = 192.168.0.1

(Dest: C0A800FE → 192.168.0.254)

### ⬛ Question 6 — Network Layer Concepts + Forwarding (15 Marks | CLO-2)

Answer the following parts:

(a) [5 marks] A router has the following forwarding table. For each destination address, state which interface is used:

| Prefix | Interface |
| --- | --- |
| 11001000 00010111 00010 | 0 |
| 11001000 00010111 00011000 | 1 |
| 11001000 00010111 00011 | 2 |
| otherwise | 3 |

- Dest A: 11001000 00010111 00011000 10101010

- Dest B: 11001000 00010111 00010110 10100001

(b) [5 marks] Explain the IP Header Checksum computation process. Why does the router recompute the checksum at every hop?

(c) [5 marks] What is the purpose of the TTL field? What problem does it solve? A datagram starts with TTL=4 and traverses 5 routers — what happens?

Complete Solutions:

(a) Longest Prefix Matching:

Dest A: 11001000 00010111 00011000 10101010

Match prefix 0:  11001000 00010111 00010...  → NO (bit 18 differs)

Match prefix 1:  11001000 00010111 00011000  → YES (24 bits match)

Match prefix 2:  11001000 00010111 00011     → YES (21 bits match)

Longest match = prefix 1 (24 bits) → Interface 1

Dest B: 11001000 00010111 00010110 10100001

Match prefix 0:  11001000 00010111 00010     → YES (21 bits match)

Match prefix 1:  11001000 00010111 00011000  → NO

Match prefix 2:  11001000 00010111 00011     → NO

Longest match = prefix 0 (21 bits) → Interface 0

(b) IP Checksum computation:

- Treat every 2 bytes of IP header as a 16-bit integer

- Sum all 16-bit integers using 1's complement arithmetic

- Take 1's complement of the result → this is the checksum

Why recompute at every hop?

Because TTL field is DECREMENTED by 1 at every router.

Since TTL changes, the header changes → old checksum is invalid.

Router must recompute and insert new checksum. (Slide 25)

(c) TTL purpose:

- Prevents datagrams from circulating forever due to routing loops

- Each router decrements TTL by 1

- When TTL = 0 → router DISCARDS the datagram

TTL=4, traverses 5 routers:

Router 1: TTL → 3

Router 2: TTL → 2

Router 3: TTL → 1

Router 4: TTL → 0 → DATAGRAM DISCARDED here

Router 5: never receives it

Datagram never reaches destination. (Slide 24)

### 📌 Quick Revision — Week 12 Ke Most Important Points

| Topic | Jo yaad rakhna ZAROOR hai |
| --- | --- |
| Forwarding | Input → Output port, single router, uses forwarding table |
| Routing | End-to-end path determination, builds routing table |
| Datagram Network | Connectionless, dest IP address, no state at routers |
| VC Network | Connection-oriented, VC numbers, state maintained |
| IP Header size | Fixed 20 bytes minimum, max 60 bytes with options |
| Fragmentation | Done by ROUTERS, Reassembly at DESTINATION ONLY |
| More flag | 0 = last fragment, 1 = more fragments follow |
| Offset units | 8 bytes (64 bits) — ALWAYS divide by 8 |
| Same ID | ALL fragments of one datagram share same Identification number |
| TTL | Decremented at each router; 0 = discard |
| Protocol field | TCP=6, UDP=17 |
| Longest Prefix | Most specific (longest) match wins |
| Checksum | Header only, recomputed at every router (because TTL changes) |

## 📋 Week 13 — Subnetting, CIDR, DHCP & NAT

### Predicted Final Term Paper — Dr. Nauman Mazhar, UCP

### ⬛ Question 1 — MCQs, T/F, Fill in Blanks (15 Marks | CLO-2)

#### Part A — MCQs (Circle correct option)

1. DHCP stands for:

- (a) Dynamic Host Connection Protocol

- (b) Dynamic Host Configuration Protocol ✅

- (c) Datagram Host Control Protocol

- (d) Dynamic Header Configuration Protocol

Answer: (b) — Slide 21

2. In DHCP, the correct sequence of messages exchanged is:

- (a) Offer → Discover → Request → ACK

- (b) Discover → Request → Offer → ACK

- (c) Discover → Offer → Request → ACK ✅

- (d) Request → Discover → Offer → ACK

Answer: (c) — Slide 21: "Discover → Offer → Request → ACK"

3. Which of the following is a valid Private IP address range used with NAT?

- (a) 197.0.0.0/24

- (b) 10.0.0.0/8 ✅

- (c) 128.119.0.0/16

- (d) 223.1.1.0/24

Answer: (b) — Slide 28: Private ranges are 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16

4. CIDR stands for:

- (a) Classless Internet Domain Routing

- (b) Classless Inter Domain Routing ✅

- (c) Class-based Internet Domain Routing

- (d) Classful Inter-Domain Routing

Answer: (b) — Slide 12

5. In subnetting, to divide a network into subnets, bits are shifted from:

- (a) NetID to HostID

- (b) HostID to NetID ✅

- (c) HostID to Protocol field

- (d) NetID to TTL field

Answer: (b) — Slide 13: "shift certain bits from HostID to NetID to make Subnet ID"

6. NAT router uses _______ to distinguish between multiple simultaneous connections sharing one public IP address:

- (a) IP addresses

- (b) MAC addresses

- (c) Port numbers ✅

- (d) TTL values

Answer: (c) — Slide 32: "16-bit port-number field: more than 60,000 simultaneous connections"

7. DHCP message is initially sent as a broadcast because:

- (a) Client already knows the DHCP server's IP address

- (b) Client has no IP address yet and does not know the server's address ✅

- (c) DHCP only works with broadcast

- (d) Router blocks unicast from new clients

Answer: (b) — Slide 23: Client IP is 0.0.0.0, uses broadcast 255.255.255.255

#### Part B — True / False

8. In a Class A network, the first byte range is 128–191. (T/F)

FALSE — Slide 7: Class A first byte = 0–127. 128–191 is Class B.

9. A /26 subnet mask means 26 bits are used for the Network/Subnet ID. (T/F)

TRUE — Slide 12: CIDR format a.b.c.d/x where x = number of bits in subnet ID

10. NAT allows multiple devices on a private network to share a single public IP address. (T/F)

TRUE — Slide 28: "allows a local N/W to connect all its devices to Internet using private IP addrs but need one/more globally unique IP addr on its router"

11. DHCP only provides an IP address to the client — nothing else. (T/F)

FALSE — Slide 24: DHCP also returns: IP of first-hop router, DNS server name/IP, and network mask.

12. The network address of a subnet has all HostID bits set to one. (T/F)

FALSE — Slide 4: Network address has all HostID bits set to zero.

#### Part C — Fill in the Blanks

13. In a Class C network, the initial byte range is _______ to _______ and each network can have a maximum of _______ hosts.

Answer: 192 to 223, 254 hosts — Slide 7

14. DHCP client uses port _______ and DHCP server uses port _______.

Answer: Client = 68, Server = 67 — Slide 23

15. The three private IPv4 address ranges are: _______, _______, and _______.

Answer: 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16 — Slide 28

### ⬛ Question 2 — Short Questions (5×5 = 25 Marks | CLO-2)

#### Q2.a

What is a Subnet? What is a Subnet Mask? How are hosts within the same subnet able to communicate without a router?

Key Points:

- Subnet: A portion of a larger network where all device interfaces share the same Subnet ID (NetID portion of IP address)

- Hosts in the same subnet can reach each other directly, without a router

- Subnet Mask (Network Prefix): Tells how many bits belong to the NetID. Written as /n (e.g., /24 means first 24 bits = NetID)

- Network Address: Subnet ID with all HostID bits = 0 (e.g., 223.1.1.0/24)

- All hosts in 223.1.1.0/24 range from 223.1.1.1 to 223.1.1.254 — same subnet, communicate directly (Slides 4, 10)

#### Q2.b

Explain DHCP with its 4-message exchange. What information does DHCP provide to a newly joining host?

Key Points:

| Step | Message | Direction | Details |
| --- | --- | --- | --- |
| 1 | DHCP Discover | Client → Broadcast | Client has no IP, broadcasts to find server |
| 2 | DHCP Offer | Server → Client | Server offers an available IP address |
| 3 | DHCP Request | Client → Broadcast | Client formally requests the offered IP |
| 4 | DHCP ACK | Server → Client | Server confirms — client can now use the IP |

DHCP provides: IP address, IP of first-hop router (default gateway), DNS server name & IP, Network mask (Slides 21–26)

#### Q2.c

What is NAT? Why was it introduced? Explain how the NAT translation table works for outgoing and incoming datagrams.

Key Points:

- Why NAT? IPv4 addresses were depleted — not enough globally unique addresses for every device

- How it works:

  - Private IPs (10.x.x.x etc.) used internally — NOT routable on Internet

  - NAT router has ONE public IP address facing the Internet

  - Outgoing: Router replaces (private IP, port) with (public IP, new port) in datagram; records mapping in NAT table

  - Incoming: Router looks up NAT table, replaces (public IP, port) back to (private IP, port), forwards internally

- Supports 60,000+ simultaneous connections using one public IP (Slides 28–32)

#### Q2.d

What is CIDR (Classless Inter Domain Routing)? How is it different from classful addressing?

Key Points:

- Classful addressing: Fixed NetID/HostID boundary (Class A=8 bits, B=16 bits, C=24 bits) — wasteful, inflexible

- CIDR: NetID can be of arbitrary length — not restricted to byte boundaries

- Format: a.b.c.d/x where x = exact number of bits in network prefix

- Example: 200.23.17.0/26 → 26 bits for network, only 6 bits for hosts (64 addresses)

- More efficient address allocation, supports subnetting at any boundary (Slide 12)

#### Q2.e

Explain IP Address Classes A, B, and C. For each, give the first byte range, number of network bits, host bits, and maximum hosts.

Key Points:

| Class | First Byte | Net bits | Host bits | Max Hosts | Use |
| --- | --- | --- | --- | --- | --- |
| A | 0 – 127 | 7 | 24 | 16,777,214 | Very large organizations |
| B | 128 – 191 | 14 | 16 | 65,534 | Large organizations |
| C | 192 – 223 | 21 | 8 | 254 | Small organizations |

- Formula: Max hosts = 2^(host bits) — 2 (subtract network addr and broadcast addr) (Slide 7)

#### Q2.f

What additional information beyond an IP address does a DHCP server provide? Why is each piece of information necessary?

Key Points:

- IP address — host's own address for communication

- First-hop router IP (default gateway) — host sends packets here when destination is on a different network

- DNS server name & IP — needed to resolve hostnames to IP addresses

- Network mask — needed to identify which part of IP address is NetID and which is HostID; helps determine if destination is on same subnet or different network (Slide 24)

### ⬛ Question 3 — Delays Numerical (15 Marks | CLO-2)

Host A sends a 4 MB file to Host B. Three links, two routers: R1 = 1 Mbps, R2 = 4 Mbps, R3 = 2 Mbps. Distances: d1 = 3000 km, d2 = 8000 km, d3 = 1000 km. Propagation speed = 3×10⁸ m/s. Router processing = 5 ms, queuing = 15 ms. Packet size = 1500 bytes. 1 MB = 10⁶ bytes.

(a) Tx delay for one packet. (b) Propagation delay for one packet. (c) End-to-end delay one packet. (d) Total time for entire file.

Complete Solution:

L = 1500 × 8 = 12,000 bits

(a) Tx Delays:

Tx1 = 12,000 / 1,000,000     = 12 ms

Tx2 = 12,000 / 4,000,000     = 3 ms

Tx3 = 12,000 / 2,000,000     = 6 ms

Total Tx                      = 21 ms

(b) Propagation Delays:

Prop1 = 3,000,000 / 3×10⁸   = 10 ms

Prop2 = 8,000,000 / 3×10⁸   = 26.67 ms

Prop3 = 1,000,000 / 3×10⁸   = 3.33 ms

Total Prop                    = 40 ms

(c) Router delays = 2 × (5+15) = 40 ms

Total (1 pkt) = 21+40+40     = 101 ms

(d) N = (4×10⁶ × 8) / 12,000 = 2667 packets

(N-1) × Tx1 = 2666 × 12 ms  = 31,992 ms

Total = 31,992 + 101         = 32,093 ms ≈ 32.09 sec

### ⬛ Question 4 — Subnetting Calculation (15 Marks | CLO-2)

An ISP has been allocated the network address block 192.168.10.0/24. The ISP needs to divide this block into the following subnets:

- (a) One subnet for 100 hosts

- (b) One subnet for 50 hosts

- (c) One subnet for 20 hosts

- (d) One subnet for 10 hosts

For each subnet, determine: subnet mask (/x), network address, first usable host IP, last usable host IP, broadcast address.

Complete Solution:

Original block: 192.168.10.0/24

Total addresses = 256 (8 host bits)

RULE: Allocate power-of-2 size ≥ required hosts + 2 (network + broadcast)

(a) 100 hosts → need 102 addresses → next power of 2 = 128 → /25

Network addr   : 192.168.10.0/25

Subnet mask    : 255.255.255.128

First host     : 192.168.10.1

Last host      : 192.168.10.126

Broadcast      : 192.168.10.127

Max hosts      : 2^7 - 2 = 126 ✓

(b) 50 hosts → need 52 addresses → next power of 2 = 64 → /26

Network addr   : 192.168.10.128/26

Subnet mask    : 255.255.255.192

First host     : 192.168.10.129

Last host      : 192.168.10.190

Broadcast      : 192.168.10.191

Max hosts      : 2^6 - 2 = 62 ✓

(c) 20 hosts → need 22 addresses → next power of 2 = 32 → /27

Network addr   : 192.168.10.192/27

Subnet mask    : 255.255.255.224

First host     : 192.168.10.193

Last host      : 192.168.10.222

Broadcast      : 192.168.10.223

Max hosts      : 2^5 - 2 = 30 ✓

(d) 10 hosts → need 12 addresses → next power of 2 = 16 → /28

Network addr   : 192.168.10.224/28

Subnet mask    : 255.255.255.240

First host     : 192.168.10.225

Last host      : 192.168.10.238

Broadcast      : 192.168.10.239

Max hosts      : 2^4 - 2 = 14 ✓

Remaining unused: 192.168.10.240 → 192.168.10.255 (16 addresses)

Verify: 128 + 64 + 32 + 16 = 240 used out of 256 ✓

### ⬛ Question 5 — NAT Translation Table (15 Marks | CLO-2)

A home network uses NAT with private IP range 192.168.1.0/24. The NAT router's public IP is 203.45.12.7. Four hosts inside the network initiate connections to external servers:

| Host (Private) | Private Port | External Server | Server Port |
| --- | --- | --- | --- |
| 192.168.1.2 | 3001 | 74.125.19.100 | 80 |
| 192.168.1.3 | 3002 | 74.125.19.100 | 80 |
| 192.168.1.4 | 3003 | 52.1.1.1 | 443 |
| 192.168.1.2 | 3004 | 8.8.8.8 | 53 |

(a) Draw the complete NAT translation table showing WAN-side and LAN-side entries.
(b) When the server 74.125.19.100 sends a reply to 203.45.12.7 port 5001, which internal host receives it?
(c) Explain the 4 steps NAT router performs when Host 192.168.1.3 sends a packet to 74.125.19.100:80.
(d) How many simultaneous connections can theoretically be supported by this single public IP?

Complete Solution:

(a) NAT Translation Table:

WAN side addr           LAN side addr

203.45.12.7, 5001  ↔   192.168.1.2, 3001

203.45.12.7, 5002  ↔   192.168.1.3, 3002

203.45.12.7, 5003  ↔   192.168.1.4, 3003

203.45.12.7, 5004  ↔   192.168.1.2, 3004

(b) Reply to 203.45.12.7:5001 → NAT table lookup →

forward to 192.168.1.2 on port 3001

(c) 4 Steps for 192.168.1.3 → 74.125.19.100:80:

Step 1: Host sends datagram:

Src=192.168.1.3:3002, Dst=74.125.19.100:80

Step 2: NAT router replaces source:

Src=203.45.12.7:5002, Dst=74.125.19.100:80

Step 3: NAT router records in translation table:

203.45.12.7:5002 ↔ 192.168.1.3:3002

Step 4: On reply (Src=74.125.19.100:80, Dst=203.45.12.7:5002):

NAT looks up table → replaces Dst to 192.168.1.3:3002

Forwards internally to 192.168.1.3

(d) Port number field = 16 bits = 2^16 = 65,536 ports

Well-known ports (0-1023) reserved

Available ≈ 65,536 - 1024 = 64,512 simultaneous connections

(Slide 32: "more than 60,000 simultaneous connections")

### ⬛ Question 6 — CIDR Block Division + IP Concepts (15 Marks | CLO-2)

Answer the following parts:

(a) [6 marks] An ISP has the block 172.16.0.0/21. Divide it into 8 equal subnets. For each subnet write the network address with prefix.

(b) [5 marks] A host has IP address 172.16.5.130/23. Determine: (i) Subnet mask in dotted decimal, (ii) Network address, (iii) Broadcast address, (iv) Maximum number of hosts, (v) Is 172.16.4.200 in the same subnet?

(c) [4 marks] Why is NAT considered controversial? Give TWO arguments against NAT from your slides.

Complete Solutions:

(a) 172.16.0.0/21 divided into 8 subnets:

/21 block = 2^(32-21) = 2048 addresses

8 subnets → each gets 2048/8 = 256 addresses → /24

Subnet 1: 172.16.0.0/24

Subnet 2: 172.16.1.0/24

Subnet 3: 172.16.2.0/24

Subnet 4: 172.16.3.0/24

Subnet 5: 172.16.4.0/24

Subnet 6: 172.16.5.0/24

Subnet 7: 172.16.6.0/24

Subnet 8: 172.16.7.0/24

(b) Host: 172.16.5.130/23

Prefix = 23 bits → host bits = 9 bits

(i) Subnet mask = 255.255.254.0

(23 bits = 11111111.11111111.11111110.00000000)

(ii) Network address: zero out last 9 bits of IP

172.16.5.130 → 172.16.4.0/23

(5 in binary = 00000101, last bit is host bit → becomes 0 in 3rd octet)

(iii) Broadcast = set all 9 host bits to 1

172.16.5.255 /23

(iv) Max hosts = 2^9 - 2 = 512 - 2 = 510

(v) 172.16.4.200 — is it in 172.16.4.0/23?

Range is 172.16.4.0 → 172.16.5.255

172.16.4.200 falls in this range → YES, same subnet ✓

(c) NAT Controversies (Slide 32):

1. "Routers should only process up to Layer 3" —

NAT modifies IP addresses and port numbers (Layer 3 & 4),

violating the principle of layer separation.

2. "Violates end-to-end argument" —

Hosts cannot communicate directly; external host cannot

initiate connection to internal host (breaks peer-to-peer).

(Bonus: Address shortage should be solved by IPv6, not NAT)

### 📌 Quick Revision — Week 13 Ke Most Important Points

| Topic | Jo yaad rakhna ZAROOR hai |
| --- | --- |
| IP Address | 32-bit, NetID + HostID, dotted decimal |
| Class A | 0–127, /8, 16M+ hosts |
| Class B | 128–191, /16, 65K hosts |
| Class C | 192–223, /24, 254 hosts |
| Subnetting | Bits move from HostID → NetID |
| CIDR | a.b.c.d/x, arbitrary prefix length |
| Network Addr | All HostID bits = 0 |
| Broadcast Addr | All HostID bits = 1 |
| Max hosts | 2^(host bits) − 2 |
| DHCP | Discover → Offer → Request → ACK |
| DHCP gives | IP + Default Gateway + DNS + Subnet Mask |
| DHCP ports | Client=68, Server=67 |
| DHCP initial | Broadcast from 0.0.0.0 to 255.255.255.255 |
| NAT | Private IPs + 1 public IP + port numbers |
| Private ranges | 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16 |
| NAT table | (public IP:port) ↔ (private IP:port) |
| NAT limit | ~60,000 simultaneous connections per public IP |

Slides parh li. Ab seedha predicted paper:

## 📋 Week 14 & 15 — Routing Algorithms: Link State (Dijkstra) & Distance Vector (Bellman-Ford)

### Predicted Final Term Paper — Dr. Nauman Mazhar, UCP

### ⬛ Question 1 — MCQs, T/F, Fill in Blanks (15 Marks | CLO-2)

#### Part A — MCQs (Circle correct option)

1. The Link State Routing Algorithm is also known as:

- (a) Bellman-Ford Algorithm

- (b) Distance Vector Algorithm

- (c) Dijkstra's Algorithm ✅

- (d) Flooding Algorithm

Answer: (c) — Slide 9: "Link-State Routing Algorithm → Dijkstra's algorithm"

2. In Link State routing, each router has knowledge of:

- (a) Only its directly connected neighbors

- (b) Complete network topology and all link costs ✅

- (c) Only the destination addresses

- (d) Distance vectors from neighbors only

Answer: (b) — Slide 9: "N/W topology, link costs known to ALL nodes"

3. The Bellman-Ford equation used in Distance Vector routing is:

- (a) dx(y) = maxv { c(x,v) + dv(y) }

- (b) dx(y) = minv { c(x,v) + dv(y) } ✅

- (c) dx(y) = minv { c(x,v) × dv(y) }

- (d) dx(y) = minv { c(x,v) − dv(y) }

Answer: (b) — Slide 14: "dx(y) = minv { c(x,v) + dv(y) }"

4. In Distance Vector routing, each node shares its distance vector with:

- (a) All nodes in the entire network

- (b) Only the destination node

- (c) Only its directly connected neighbors ✅

- (d) A central routing server

Answer: (c) — Slide 16: "each node sends its own DV to neighbors"

5. Link cost can be defined in terms of which of the following? (Select most complete answer)

- (a) Number of hops only

- (b) Bandwidth only

- (c) Number of hops, congestion, price, or bandwidth ✅

- (d) IP address and MAC address

Answer: (c) — Slide 6: "Link cost in terms of: num of hops, congestion, price, bandwidth"

6. In Dijkstra's algorithm, after k iterations, the algorithm knows:

- (a) Paths to all nodes in the network

- (b) Only paths to neighbor nodes

- (c) Least cost paths to k destinations ✅

- (d) Nothing useful until all iterations complete

Answer: (c) — Slide 9: "after k iterations, knows least cost paths to k destinations"

7. Distance Vector algorithm is described as "distributed" because:

- (a) It runs on a central server and distributes results

- (b) Each node receives neighbor info, performs calculations independently, then distributes results back to neighbors ✅

- (c) It broadcasts to all nodes simultaneously

- (d) It requires all nodes to operate in lock-step manner

Answer: (b) — Slide 21: "each node receives neighbor info, performs calculations independently, then distributes results back to neighbors"

#### Part B — True / False

8. In Link State routing, routers share their distance vectors with all other routers in the network. (T/F)

FALSE — Slide 7: Link State uses "link state broadcast" (topology info). Distance Vectors are shared in DV routing. LS routers share link STATE, not distance vectors.

9. Dijkstra's algorithm is an iterative algorithm — each iteration adds one more node to the set N' of nodes with known least-cost paths. (T/F)

TRUE — Slide 9: "iterative: after k iterations, knows least cost paths to k destinations"

10. Distance Vector algorithm requires all nodes to perform calculations at exactly the same time (synchronously). (T/F)

FALSE — Slide 21: "asynchronous: does not require all nodes to operate in lock-step manner"

11. In a graph representation of a network, routers are represented as nodes and links are represented as edges with costs. (T/F)

TRUE — Slide 5: "routers are nodes, links are edges with costs"

12. The cost of a path is the maximum link cost along that path. (T/F)

FALSE — Slide 6: "Cost of Path = sum of costs of all links that form the path"

#### Part C — Fill in the Blanks

13. In Dijkstra's algorithm, N' represents _______.

Answer: The set of nodes whose least cost paths have already been computed — Slide 9

14. In the DV algorithm notation, c(x,v) represents _______ and dv(y) represents _______.

Answer: c(x,v) = cost of link from node x to neighbor v; dv(y) = cost of path from neighbor v to destination y — Slide 14

15. The two main classifications of routing algorithms based on information scope are _______ and _______.

Answer: Global (Link State) and Decentralized (Distance Vector) — Slide 7

### ⬛ Question 2 — Short Questions (5×5 = 25 Marks | CLO-2)

#### Q2.a

What is Routing? What are the main components involved in routing?

Key Points:

- Routing = "process of selecting optimal paths in a network along which to send data packets" (Slide 3)

- Goal: determine least cost paths for packets from source to all destinations — ensuring efficient, reliable, timely delivery

- Components:

  - Routers — physical devices that forward packets

  - Routing Protocols/Algorithms — compute optimal paths (Dijkstra, Bellman-Ford, RIP, OSPF, BGP)

  - Routing Tables — store computed paths; used to populate forwarding tables

- Routing algorithm runs → builds routing table → populates forwarding table → forwarding uses forwarding table per packet

#### Q2.b

Compare Link State and Distance Vector routing algorithms on at least FOUR points.

Key Points:

| Feature | Link State (Dijkstra) | Distance Vector (Bellman-Ford) |
| --- | --- | --- |
| Info known to each router | Complete network topology + all link costs | Only neighbor distances (DV) |
| What is shared | Link state info broadcast to ALL nodes | Distance vectors shared only with direct neighbors |
| Algorithm used | Dijkstra's shortest path algorithm | Bellman-Ford equation |
| Computation | Each node computes full routing table locally | Iterative, distributed — each node updates based on neighbors |
| Info scope | Global | Decentralized |
| Convergence | Faster (complete info) | Slower (iterative exchange) |
| Scalability | Less scalable (large broadcasts) | More scalable for large networks |

(Slides 7, 9, 14)

#### Q2.c

Explain the Bellman-Ford equation. Using a simple example with 3 nodes, demonstrate how it computes the shortest path.

Key Points:

- Equation: dx(y) = minv { c(x,v) + dv(y) }

- Meaning: The least cost from x to y = minimum over all neighbors v of (cost to reach v + v's cost to reach y)

- The neighbor v that achieves the minimum becomes the next hop in the forwarding table

Example (from Slide 15):

Nodes: u, x, v, w, z

Find du(z):

c(u,x) = 1, dx(z) = 3  → 1+3 = 4

c(u,v) = 2, dv(z) = 5  → 2+5 = 7

c(u,w) = 5, dw(z) = 3  → 5+3 = 8

du(z) = min{4, 7, 8} = 4  → via x (next hop = x)

#### Q2.d

Describe the three key properties of the Distance Vector Algorithm: Iterative, Asynchronous, and Distributed.

Key Points: (Slide 21)

- Iterative: Algorithm runs in steps. Each node waits for info from neighbors, recomputes, sends updates. Continues until no more changes — i.e., algorithm converges

- Asynchronous: Nodes do NOT need to operate in lock-step. Each node updates at its own pace when it receives new info from a neighbor. No global clock or synchronization needed

- Distributed: Each node receives neighbor info, performs its OWN calculations independently, then sends results back to its neighbors. No central controller needed.

#### Q2.e

How does the DV algorithm react when a link cost changes? Describe the "wait → recompute → notify" cycle.

Key Points: (Slide 21)

- Each node continuously runs this loop:

  - WAIT — for a change in local link cost OR a new DV message from a neighbor

  - RECOMPUTE — apply Bellman-Ford equation to update own distance vector

  - NOTIFY — if DV to any destination has changed, send updated DV to all neighbors

- Neighbors then repeat the same cycle → eventually the whole network converges to correct routes

- Good news travels fast — when link cost decreases, updates propagate quickly

- Bad news travels slowly — when link cost increases, "count to infinity" problem can occur

#### Q2.f

What is graph abstraction in routing? Define: Node, Edge, Link Cost, Path Cost.

Key Points: (Slides 5–6)

- Network is modeled as a graph G = (N, E)

- N (Nodes) = set of routers e.g., {u, v, w, x, y, z}

- E (Edges) = set of links between routers e.g., {(u,v), (u,x), ...}

- c(x,y) = cost of link between directly connected nodes x and y. If not directly connected, c(x,y) = ∞

- Path Cost = sum of all link costs along the path: c(x1,x2) + c(x2,x3) + ... + c(xp-1,xp)

- Link cost can represent: hops (directly), congestion (directly), price (directly), bandwidth (inversely)

- Routing algorithm finds least cost path between any two nodes

### ⬛ Question 3 — Delays Numerical (15 Marks | CLO-2)

Host A sends a 2 MB file to Host B over 3 links with 2 routers. R1 = 500 Kbps, R2 = 3 Mbps, R3 = 1 Mbps. Distances: d1 = 6000 km, d2 = 200 km, d3 = 1500 km. Prop speed = 2.5×10⁸ m/s. Router delays: processing = 3 ms, queuing = 7 ms. Packet = 1000 bytes. 1 MB = 10⁶ bytes.

(a) Tx delay — one packet. (b) Prop delay — one packet. (c) End-to-end delay — one packet. (d) Total time — entire file.

Complete Solution:

L = 1000 × 8 = 8000 bits

(a) Transmission Delays:

Tx1 = 8000 / 500,000          = 16 ms

Tx2 = 8000 / 3,000,000        = 2.67 ms

Tx3 = 8000 / 1,000,000        = 8 ms

Total Tx                       = 26.67 ms

(b) Propagation Delays:

Prop1 = 6,000,000 / 2.5×10⁸  = 24 ms

Prop2 = 200,000 / 2.5×10⁸    = 0.8 ms

Prop3 = 1,500,000 / 2.5×10⁸  = 6 ms

Total Prop                     = 30.8 ms

(c) Router delays = 2 × (3+7)  = 20 ms

Total (1 pkt) = 26.67+30.8+20 = 77.47 ms

(d) N = (2×10⁶ × 8) / 8000    = 2000 packets

(N−1) × Tx1 = 1999 × 16      = 31,984 ms

Total = 31,984 + 77.47        = 32,061.47 ms

≈ 32.06 seconds

### ⬛ Question 4 — Dijkstra's Algorithm Full Table (15 Marks | CLO-2)

Apply Dijkstra's algorithm to find the least cost paths from node u to ALL other nodes in the following network:

Nodes: u, v, w, x, y, z

Link costs:

c(u,v) = 2    c(u,w) = 5    c(u,x) = 1

c(v,w) = 3    c(v,x) = 2

c(w,x) = 3    c(w,y) = 1    c(w,z) = 5

c(x,y) = 1

c(y,z) = 2

(a) Fill in the complete Dijkstra iteration table step by step.
(b) Write the final least cost from u to each node.
(c) Determine the next hop for each destination in u's forwarding table.

Complete Solution:

Initial: N' = {u}

D(v)=2,p(v)=u   D(w)=5,p(w)=u   D(x)=1,p(x)=u   D(y)=∞   D(z)=∞

NOTATION: D(node) = current best cost, p(node) = predecessor

Step 0 — Initialize:

N' = {u}

D(v)=2  D(w)=5  D(x)=1  D(y)=∞  D(z)=∞

Step 1 — Add x (smallest D = 1):

N' = {u, x}

Update neighbors of x:

D(v) = min{2, D(x)+c(x,v)} = min{2, 1+2} = 2  p(v)=u (no change)

D(w) = min{5, D(x)+c(x,w)} = min{5, 1+3} = 4  p(w)=x  ← updated

D(y) = min{∞, D(x)+c(x,y)} = min{∞, 1+1} = 2  p(y)=x  ← updated

Step 2 — Add v or y (both D=2, pick v):

N' = {u, x, v}

Update neighbors of v:

D(w) = min{4, D(v)+c(v,w)} = min{4, 2+3} = 4  p(w)=x (no change)

D(y) = min{2, D(v)+c(v,y)} = no direct link  (no change)

Step 3 — Add y (D=2):

N' = {u, x, v, y}

Update neighbors of y:

D(w) = min{4, D(y)+c(y,w)} = min{4, 2+1} = 3  p(w)=y  ← updated

D(z) = min{∞, D(y)+c(y,z)} = min{∞, 2+2} = 4  p(z)=y  ← updated

Step 4 — Add w (D=3):

N' = {u, x, v, y, w}

Update neighbors of w:

D(z) = min{4, D(w)+c(w,z)} = min{4, 3+5} = 4  p(z)=y (no change)

Step 5 — Add z (D=4):

N' = {u, x, v, y, w, z}  ← ALL NODES DONE

FINAL RESULTS TABLE:

╔══════╦══════════╦═════════════╦══════════╗

║ Dest ║ Min Cost ║ Path        ║ Next Hop ║

╠══════╬══════════╬═════════════╬══════════╣

║  v   ║    2     ║ u → v       ║    v     ║

║  w   ║    3     ║ u→x→y→w    ║    x     ║

║  x   ║    1     ║ u → x       ║    x     ║

║  y   ║    2     ║ u → x → y  ║    x     ║

║  z   ║    4     ║ u→x→y→z    ║    x     ║

╚══════╩══════════╩═════════════╩══════════╝

(c) Forwarding Table at u:

Destination  →  Output Link (Next Hop)

v        →       v

w        →       x

x        →       x

y        →       x

z        →       x

### ⬛ Question 5 — Bellman-Ford / DV Algorithm (15 Marks | CLO-2)

Three nodes x, y, z with the following link costs:

c(x,y) = 2    c(y,z) = 1    c(x,z) = 7

(a) Write the initial distance vector table for ALL three nodes (time t=0).
(b) After nodes exchange distance vectors once (t=1), compute the updated distance vector for each node using the Bellman-Ford equation. Show all calculations.
(c) After the second exchange (t=2), show the final converged distance vectors.
(d) What is the least cost path from x to z? Through which node does it pass?

Complete Solution:

(a) Initial Distance Vectors (t=0) — each node only knows direct link costs:

Node x knows: dx(x)=0, dx(y)=2, dx(z)=7

Node y knows: dy(x)=2, dy(y)=0, dy(z)=1

Node z knows: dz(x)=7, dz(y)=1, dz(z)=0

DV Tables at t=0:

cost to

x    y    z

from x: 0    2    7

from y: 2    0    1

from z: 7    1    0

(b) After first exchange (t=1) — each node receives neighbors' DVs:

Node x updates using B-F: dx(y) = minv{c(x,v)+dv(y)}

dx(y) = min{c(x,y)+dy(y), c(x,z)+dz(y)}

= min{2+0,  7+1}  = min{2, 8} = 2  (no change)

dx(z) = min{c(x,y)+dy(z), c(x,z)+dz(z)}

= min{2+1,  7+0}  = min{3, 7} = 3  ← UPDATED via y!

Node y updates:

dy(x) = min{c(y,x)+dx(x), c(y,z)+dz(x)}

= min{2+0, 1+7} = min{2,8} = 2 (no change)

dy(z) = min{c(y,x)+dx(z), c(y,z)+dz(z)}

= min{2+7, 1+0} = min{9,1} = 1 (no change)

Node z updates:

dz(x) = min{c(z,y)+dy(x), c(z,x)+dx(x)}

= min{1+2, 7+0} = min{3,7} = 3  ← UPDATED via y!

dz(y) = min{c(z,y)+dy(y), c(z,x)+dx(y)}

= min{1+0, 7+2} = min{1,9} = 1 (no change)

DV Tables at t=1:

cost to

x    y    z

from x: 0    2    3   ← z cost updated: 7→3

from y: 2    0    1

from z: 3    1    0   ← x cost updated: 7→3

(c) t=2: nodes exchange again — no further changes → CONVERGED

Final DV Tables (converged):

cost to

x    y    z

from x: 0    2    3

from y: 2    0    1

from z: 3    1    0

(d) Least cost from x to z = 3

Path: x → y → z

(cost = c(x,y) + c(y,z) = 2 + 1 = 3)

Direct x→z costs 7, but via y costs only 3

Next hop in x's forwarding table for z = y

### ⬛ Question 6 — Dijkstra on New Graph + Concepts (15 Marks | CLO-2)

Answer the following:

(a) [8 marks] Apply Dijkstra's algorithm from source node A on the following graph. Find shortest paths to ALL nodes. Show the complete iteration table.

Nodes: A, B, C, D, E

c(A,B)=4  c(A,C)=2

c(B,C)=1  c(B,D)=5

c(C,D)=8  c(C,E)=10

c(D,E)=2

(b) [4 marks] What is the difference between Static and Dynamic routing algorithms?

(c) [3 marks] Define: (i) N' in Dijkstra's algorithm (ii) D(v) (iii) p(v)

Complete Solutions:

(a) Dijkstra from A:

Step 0 — Initialize:

N'={A}, D(B)=4,p(B)=A  D(C)=2,p(C)=A  D(D)=∞  D(E)=∞

Step 1 — Add C (min D=2):

N'={A,C}

D(B)=min{4, 2+1}=3  p(B)=C  ← updated

D(D)=min{∞, 2+8}=10 p(D)=C

D(E)=min{∞, 2+10}=12 p(E)=C

Step 2 — Add B (min D=3):

N'={A,C,B}

D(D)=min{10, 3+5}=8  p(D)=B  ← updated

D(E)=min{12, 3+∞}=12 (no B-E link)

Step 3 — Add D (min D=8):

N'={A,C,B,D}

D(E)=min{12, 8+2}=10 p(E)=D  ← updated

Step 4 — Add E (min D=10):

N'={A,C,B,D,E} ← DONE

FINAL TABLE:

Dest │ Cost │ Path

B  │  3   │ A→C→B

C  │  2   │ A→C

D  │  8   │ A→C→B→D

E  │  10  │ A→C→B→D→E

(b) Static vs Dynamic Routing:

Static:

- Routes remain unchanged or change very slowly

- Admin manually configures routes

- Updates are infrequent

- Simple but cannot adapt to network changes

Dynamic:

- Routes change quickly in response to N/W traffic or topology changes

- Automatic updates via routing protocols (RIP, OSPF)

- Periodic updates between routers

- More complex but adapts to failures and congestion

(Slide 7)

(c) Dijkstra Notation:

N' = Set of nodes whose LEAST COST PATHS from source have been definitely computed

D(v) = Current estimate of least cost path from source node to destination v

p(v) = Previous node (predecessor/neighbor of v) along the current least cost path from source to v

### 📌 Quick Revision — Week 14 & 15 Most Important Points

| Topic | Yaad rakhna ZAROOR hai |
| --- | --- |
| Routing definition | Selecting optimal paths to send packets across network |
| Graph G=(N,E) | N=routers, E=links with costs |
| Path cost | SUM of all link costs (not max, not min) |
| Link cost | hops/congestion/price (directly), bandwidth (inversely) |
| LS Algorithm | Dijkstra — needs COMPLETE topology — broadcast link states |
| DV Algorithm | Bellman-Ford — needs only NEIGHBOR info — share DVs only with neighbors |
| N' in Dijkstra | Set of nodes with CONFIRMED least cost paths |
| D(v) | Current best estimated cost to node v |
| p(v) | Previous node on path to v |
| Bellman-Ford | dx(y) = minv { c(x,v) + dv(y) } |
| Next hop | The neighbor v that achieves minimum in B-F equation |
| DV properties | Iterative + Asynchronous + Distributed |
| DV convergence | Nodes exchange DVs repeatedly until no more changes |
| Static routing | Manual, slow to update |
| Dynamic routing | Automatic, reacts to topology/traffic changes |
| LS vs DV scope | LS = Global, DV = Decentralized |

Slides parh li. Seedha predicted paper:

## 📋 Week 16 & 17 — Data Link Layer, MAC Address, ARP & A Day in the Life

### Predicted Final Term Paper — Dr. Nauman Mazhar, UCP

### ⬛ Question 1 — MCQs, T/F, Fill in Blanks (15 Marks | CLO-2)

#### Part A — MCQs (Circle correct option)

1. The Data Link Layer is responsible for transporting frames between:

- (a) Source host and destination host across the entire Internet

- (b) Physically adjacent nodes over a single link ✅

- (c) Application processes on different hosts

- (d) Routers using IP addresses

Answer: (b) — Slide 3: "transport link layer frames between physically adjacent nodes over a link"

2. A MAC address is how many bits long?

- (a) 32 bits

- (b) 64 bits

- (c) 48 bits ✅

- (d) 128 bits

Answer: (c) — Slide 11: "48 bit MAC address — Ethernet or WiFi address"

3. In ARP, when a node does not know the MAC address of the target, it sends an ARP Query to:

- (a) The default router only

- (b) The DNS server

- (c) All nodes on the LAN using broadcast address FF-FF-FF-FF-FF-FF ✅

- (d) Directly to the target node's IP address

Answer: (c) — Slide 14: "A broadcasts ARP Query pkt... dest MAC addr FF-FF-FF-FF-FF-FF"

4. When Host A sends a datagram to Host B on a different LAN via router R, the destination MAC address in the frame created by A is:

- (a) B's MAC address

- (b) R's MAC address on A's side ✅

- (c) FF-FF-FF-FF-FF-FF (broadcast)

- (d) A's own MAC address

Answer: (b) — Slide 16: "A creates link-layer frame with R's MAC address as dest"

5. ARP table entries have a TTL field. What happens when TTL expires?

- (a) The entry is refreshed automatically

- (b) The entry is deleted (soft state — times out unless refreshed) ✅

- (c) A new ARP broadcast is sent immediately

- (d) The IP address changes

Answer: (b) — Slide 14: "A caches IP-to-MAC addr pair until info becomes old (times out) — soft state"

6. Which of the following is a Random Access Multiple Access protocol?

- (a) TDM

- (b) FDM

- (c) Token Ring

- (d) CSMA/CD ✅

Answer: (d) — Slide 9: Random access protocols include CSMA/CD (Ethernet uses it)

7. In "A Day in the Life of a Web Request", the FIRST protocol used by a newly connected laptop is:

- (a) DNS

- (b) HTTP

- (c) DHCP ✅

- (d) ARP

Answer: (c) — Slide 24: "connecting laptop needs to get its own IP address... use DHCP" — DHCP is first

#### Part B — True / False

8. IP addresses are used to move frames between nodes on the SAME LAN. (T/F)

FALSE — Slide 11: MAC addresses are used for same-LAN communication. IP addresses are used for network-layer (Layer 3) forwarding/routing.

9. ARP is "plug-and-play" — nodes create ARP tables automatically without network admin help. (T/F)

TRUE — Slide 14: "ARP is plug-and-play: nodes create ARP tables without help from network admins"

10. When routing to another LAN, the IP source and destination addresses in the datagram change at each router hop. (T/F)

FALSE — Slides 16–19: IP source (A) and destination (B) remain the SAME throughout. Only the MAC addresses in the frame change at each hop.

11. A link-layer frame encapsulates an IP datagram inside it. (T/F)

TRUE — Slide 3: "Frames encapsulate IP datagram"

12. In ARP, the target node replies to the ARP query using a broadcast. (T/F)

FALSE — Slide 14: "B unicasts ARP Reply pkt, gives own MAC addr — frame sent to MAC addr of A." Query is broadcast; Reply is unicast.

#### Part C — Fill in the Blanks

13. The ARP table stores entries in the format: < _______, _______, _______ >

Answer: < IP addr ; MAC addr ; TTL > — Slide 13

14. In the protocol stack, a link-layer packet is called a _______, while a network-layer packet is called a _______.

Answer: Frame (link layer), Datagram (network layer) — Slides 3–5

15. The three broad classes of multiple access protocols are: _______, _______, and _______.

Answer: Channel Partitioning, Taking Turns, Random Access — Slide 9

### ⬛ Question 2 — Short Questions (5×5 = 25 Marks | CLO-2)

#### Q2.a

What is the Data Link Layer? What is its main responsibility? How does it relate to the Network Layer?

Key Points:

- Nodes = hosts & routers. Links = comm channels connecting adjacent nodes. Frames = Layer-2 packets

- DLL Responsibility: Transport link-layer frames between physically adjacent nodes over a single link (Slide 3)

- Relationship with Network Layer:

  - Network Layer sends IP datagram from source to destination host (end-to-end)

  - Link Layer carries IP datagram inside a frame across each individual link (Slide 4)

  - Different links can use different link-layer protocols (Ethernet, Frame Relay, WiFi)

- Encapsulation: Message → Segment → Datagram → Frame → Bits (Slide 5)

#### Q2.b

What is a MAC address? How is it different from an IP address? Why are both needed?

Key Points:

| Feature | MAC Address | IP Address |
| --- | --- | --- |
| Size | 48 bits (6 bytes) | 32 bits (4 bytes) |
| Layer | Layer 2 (Data Link) | Layer 3 (Network) |
| Written as | 1A-2F-BB-76-09-AD | 192.168.1.1 |
| Scope | Same LAN only | End-to-end, entire Internet |
| Assigned by | Manufacturer (burned in NIC) | Network admin or DHCP |
| Changes? | Never (permanent) | Changes with network |

- Why both? MAC address moves frame across ONE link. IP address routes datagram across the ENTIRE path from source to destination. They work together at each hop. (Slides 11–12)

#### Q2.c

Explain the ARP protocol. What problem does it solve? Describe the complete ARP request-reply process.

Key Points:

- Problem: When Host A wants to send a frame to Host B on the same LAN, A needs B's MAC address. A only knows B's IP address. ARP resolves IP → MAC.

- ARP Table: Each node maintains table of <IP addr, MAC addr, TTL> mappings (Slide 13)

- Process when MAC not in table: (Slide 14)

  - A broadcasts ARP Query containing B's IP address to FF-FF-FF-FF-FF-FF

  - ALL nodes on LAN receive the query

  - Only B recognizes its own IP → B sends ARP Reply (unicast) with its MAC address back to A

  - A caches the mapping in its ARP table (with TTL)

- Soft state: Entry times out unless refreshed — prevents stale entries

#### Q2.d

Trace the complete step-by-step process when Host A (on one LAN) sends a datagram to Host B (on a different LAN) via Router R. Show what changes and what stays the same at each hop.

Key Points: (Slides 15–20)

| Hop | Frame Src MAC | Frame Dst MAC | IP Src | IP Dst |
| --- | --- | --- | --- | --- |
| A → R | A's MAC | R's MAC (LAN1 side) | A's IP | B's IP |
| R → B | R's MAC (LAN2 side) | B's MAC | A's IP | B's IP |

- IP addresses NEVER change — A's IP remains source, B's IP remains destination throughout

- MAC addresses change at every hop — each router replaces MAC addresses for the next link

- How does A know R's MAC? → A uses ARP to find R's MAC

- How does R know B's MAC? → R uses ARP on B's LAN to find B's MAC

#### Q2.e

Describe "A Day in the Life of a Web Request." List all the protocols used in the correct sequence when a student's laptop connects to the campus network and requests www.google.com.

Key Points: (Slides 22–29)

| Step | Protocol | Purpose |
| --- | --- | --- |
| 1 | DHCP | Laptop gets IP address, default gateway, DNS server IP |
| 2 | ARP | Laptop finds MAC address of first-hop router |
| 3 | DNS | Laptop resolves www.google.com to IP address |
| 4 | ARP again | (if needed for routing to DNS) |
| 5 | TCP (SYN) | 3-way handshake with Google web server |
| 6 | HTTP | HTTP GET request sent, web page received |

- DHCP message is encapsulated in UDP → IP → Ethernet frame → broadcast

- DNS query is encapsulated in UDP → IP → Ethernet frame → via router

- HTTP runs over TCP — must establish connection first

#### Q2.f

What are Multiple Access Protocols? Why are they needed? Name and briefly describe the THREE types.

Key Points: (Slides 8–9)

- Why needed? When two or more nodes transmit on a shared broadcast channel simultaneously → collision → interference → data corrupted

- Multiple Access Protocol = distributed algorithm to determine which node transmits, when, and for how long

Three Types:

- Channel Partitioning: Divide channel into pieces (time slots = TDM, frequency bands = FDM). Allocate dedicated piece to each node. No collision but wasteful when node has nothing to send.

- Taking Turns: Nodes take turns (token passing, polling). More orderly. Nodes with more data get longer turns.

- Random Access: Channel NOT divided. Nodes transmit whenever they want → collisions can occur → protocol detects and recovers from collisions (CSMA/CD used in Ethernet).

### ⬛ Question 3 — Delays Numerical (15 Marks | CLO-2)

A student laptop on a campus network sends a 3 MB file to a server. Path: 3 links, 2 routers. R1 = 100 Mbps (Ethernet LAN), R2 = 10 Mbps (WAN), R3 = 1 Gbps (server LAN). Distances: d1 = 100 m, d2 = 500 km, d3 = 50 m. Prop speed = 2×10⁸ m/s. Router processing = 1 ms, queuing = 5 ms. Packet size = 1500 bytes. 1 MB = 10⁶ bytes.

(a) Tx delay — one packet.
(b) Prop delay — one packet.
(c) End-to-end delay — one packet.
(d) Total time for entire 3 MB file.

Complete Solution:

L = 1500 × 8 = 12,000 bits

(a) Transmission Delays:

Tx1 = 12,000 / 100,000,000    = 0.12 ms

Tx2 = 12,000 / 10,000,000     = 1.2 ms

Tx3 = 12,000 / 1,000,000,000  = 0.012 ms

Total Tx                       = 1.332 ms

(b) Propagation Delays:

Prop1 = 100 / 2×10⁸           = 0.0005 ms ≈ 0 ms

Prop2 = 500,000 / 2×10⁸       = 2.5 ms

Prop3 = 50 / 2×10⁸            = 0.00025 ms ≈ 0 ms

Total Prop                     = 2.5 ms

(c) Router delays = 2 × (1+5)  = 12 ms

Total (1 pkt) = 1.332+2.5+12  = 15.832 ms

(d) N = (3×10⁶ × 8) / 12,000  = 2000 packets

(N−1) × Tx1 = 1999 × 0.12    = 239.88 ms

Total = 239.88 + 15.832       = 255.71 ms ≈ 0.256 sec

### ⬛ Question 4 — ARP Process + Routing to Another LAN (15 Marks | CLO-2)

Consider the following network setup:

LAN 1 (111.111.111.0/24):

Host A:  IP=111.111.111.111,  MAC=AA-AA-AA-AA-AA-AA

Host C:  IP=111.111.111.100,  MAC=CC-CC-CC-CC-CC-CC

Router R (LAN1 side): IP=111.111.111.112, MAC=BB-BB-BB-BB-BB-BB

LAN 2 (222.222.222.0/24):

Host B:  IP=222.222.222.222,  MAC=DD-DD-DD-DD-DD-DD

Router R (LAN2 side): IP=222.222.222.220, MAC=EE-EE-EE-EE-EE-EE

Host A wants to send a datagram to Host B.

(a) A does not have R's MAC address. Describe the complete ARP process A uses to find R's MAC.

(b) Fill in the frame and datagram fields when A sends the data to Router R:

| Field | Value |
| --- | --- |
| Frame Source MAC | ? |
| Frame Dest MAC | ? |
| IP Source | ? |
| IP Destination | ? |

(c) Fill in the frame and datagram fields when Router R forwards to Host B:

| Field | Value |
| --- | --- |
| Frame Source MAC | ? |
| Frame Dest MAC | ? |
| IP Source | ? |
| IP Destination | ? |

(d) When does Host C receive any of these frames? Why?

Complete Solution:

(a) ARP Process for A to find R's MAC:

Step 1: A checks ARP table → R's MAC not found

Step 2: A broadcasts ARP Query:

Src IP=111.111.111.111, Src MAC=AA-AA-AA-AA-AA-AA

Target IP=111.111.111.112

Dest MAC=FF-FF-FF-FF-FF-FF (broadcast)

Step 3: ALL nodes on LAN1 receive: A, C, and Router R

Step 4: Only R recognizes its own IP (111.111.111.112)

Step 5: R sends unicast ARP Reply to A:

"My MAC is BB-BB-BB-BB-BB-BB"

Step 6: A updates ARP table:

111.111.111.112 → BB-BB-BB-BB-BB-BB (with TTL)

(b) Frame from A → Router R:

Frame Source MAC  : AA-AA-AA-AA-AA-AA  (A's MAC)

Frame Dest MAC    : BB-BB-BB-BB-BB-BB  (R's LAN1 MAC)

IP Source         : 111.111.111.111    (A's IP — never changes)

IP Destination    : 222.222.222.222    (B's IP — never changes)

(c) Frame from Router R → Host B:

Frame Source MAC  : EE-EE-EE-EE-EE-EE  (R's LAN2 MAC)

Frame Dest MAC    : DD-DD-DD-DD-DD-DD  (B's MAC)

IP Source         : 111.111.111.111    (still A's IP)

IP Destination    : 222.222.222.222    (still B's IP)

KEY POINT: IP addresses stay the same end-to-end.

Only MAC addresses change at each hop.

(d) Host C receives the ARP Query broadcast (Step 2)

because it is on LAN1 and all nodes receive broadcasts.

C does NOT respond because target IP ≠ C's IP.

C does NOT receive the data frames because those

frames have MAC dest = BB-BB-BB-BB-BB-BB (R's MAC),

not C's MAC. C's NIC discards them.

### ⬛ Question 5 — Day in the Life Full Trace (15 Marks | CLO-3)

A student connects a new laptop to a campus network. Trace the complete sequence of events and protocols used to load www.google.com in the browser. For each step state: protocol name, what is sent, and what information is obtained.

Complete Solution:

STEP 1 — GET AN IP ADDRESS: DHCP

What happens:

- Laptop has no IP yet → IP = 0.0.0.0

- Laptop broadcasts DHCP Discover (UDP:68 → 255.255.255.255:67)

- Encapsulated: DHCP → UDP → IP → Ethernet frame

- Ethernet broadcast: dest MAC = FF:FF:FF:FF:FF:FF

- Router (DHCP server) replies with DHCP Offer

- Laptop sends DHCP Request, server sends DHCP ACK

Information obtained:

✓ Own IP address (e.g., 68.80.2.101)

✓ IP of first-hop router (default gateway)

✓ IP and name of DNS server

✓ Subnet mask

STEP 2 — FIND ROUTER'S MAC: ARP

What happens:

- Laptop knows router's IP (from DHCP) but not MAC

- Laptop broadcasts ARP Query: "Who has router's IP?"

- Router sends unicast ARP Reply with its MAC address

Information obtained:

✓ MAC address of first-hop router

STEP 3 — RESOLVE HOSTNAME: DNS

What happens:

- Browser needs IP of www.google.com

- DNS Query created → UDP → IP → Ethernet frame

- Frame sent to router (using router's MAC from step 2)

- Router forwards IP datagram through Comcast network to DNS server

- DNS server replies with IP address of www.google.com

Information obtained:

✓ IP address of www.google.com (e.g., 64.233.169.105)

STEP 4 — ESTABLISH TCP CONNECTION: TCP 3-Way Handshake

What happens:

- Client sends TCP SYN to Google's web server IP

- SYN routed through Comcast network to Google's server

- Server replies with SYN-ACK

- Client sends ACK → TCP connection ESTABLISHED

Information obtained:

✓ Reliable, ordered TCP connection ready

STEP 5 — REQUEST WEB PAGE: HTTP

What happens:

- Client sends HTTP GET request through TCP socket

- IP datagram routed to www.google.com

- Web server sends HTTP Reply containing HTML of web page

- IP datagram with HTTP reply routed back to client

Information obtained:

✓ Web page HTML content → displayed in browser

SUMMARY PROTOCOL ORDER:

DHCP → ARP → DNS → TCP (SYN/SYN-ACK/ACK) → HTTP GET → HTTP Reply

### ⬛ Question 6 — Link Layer Concepts + MAC/ARP Combined (15 Marks | CLO-2)

Answer the following parts:

(a) [5 marks] Explain the difference between Point-to-Point links and Broadcast links. Give two examples of each. Why do broadcast links need multiple access protocols?

(b) [5 marks] A node's ARP table has the following entries:

| IP Address | MAC Address | TTL remaining |
| --- | --- | --- |
| 10.0.0.1 | AA-BB-CC-DD-EE-01 | 900 sec |
| 10.0.0.2 | AA-BB-CC-DD-EE-02 | 5 sec |
| 10.0.0.3 | AA-BB-CC-DD-EE-03 | 0 sec |

The node wants to send frames to: (i) 10.0.0.1, (ii) 10.0.0.2, (iii) 10.0.0.3. For each, describe what happens.

(c) [5 marks] Explain encapsulation at the link layer. When a laptop sends an HTTP GET to a web server, show the full encapsulation stack (all 4 layers) with the protocol data unit name at each layer.

Complete Solutions:

(a) Point-to-Point vs Broadcast Links:

Point-to-Point:

- Single sender and single receiver on the link

- Examples: PPP (dial-up modem), dedicated WAN links

between routers, Ethernet switch-to-host link

- No need for multiple access protocol (only 2 nodes)

Broadcast:

- Multiple nodes connected to SAME shared medium

- ALL nodes receive any frame transmitted

- Examples: Old Ethernet (coaxial bus), 802.11 WiFi LAN,

upstream HFC (cable TV network)

- NEED multiple access protocol because if 2+ nodes

transmit simultaneously → collision → data lost

(Slide 7–8)

(b) ARP Table Lookup:

(i) 10.0.0.1 — TTL=900sec (valid):

MAC found in table: AA-BB-CC-DD-EE-01

Node directly uses this MAC, no ARP needed.

(ii) 10.0.0.2 — TTL=5sec (valid but expiring soon):

MAC found in table: AA-BB-CC-DD-EE-02

Node uses this MAC now. In 5 seconds entry expires

and next frame will require a new ARP query.

(iii) 10.0.0.3 — TTL=0sec (expired = soft state timed out):

Entry is considered stale/invalid.

Node must send ARP broadcast query:

"Who has IP 10.0.0.3? Tell me your MAC."

Wait for ARP reply, then cache new entry, then send frame.

(c) Encapsulation Stack (HTTP GET going down):

Application Layer:

PDU = MESSAGE (HTTP GET request)

Transport Layer (TCP):

PDU = SEGMENT

Adds: Src port, Dest port (80), Seq#, ACK#, flags...

Network Layer (IP):

PDU = DATAGRAM

Adds: Src IP, Dest IP, TTL, Protocol=6(TCP)...

Link Layer (Ethernet):

PDU = FRAME

Adds: Src MAC, Dest MAC (next-hop router's MAC), EtherType...

Physical Layer:

PDU = BITS

Frame converted to electrical/optical/radio signals

Stack going UP at receiver: Bits→Frame→Datagram→Segment→Message

Each layer strips its own header and passes payload UP.

### 📌 Quick Revision — Week 16 & 17 Most Important Points

| Topic | Yaad rakhna ZAROOR hai |
| --- | --- |
| Link Layer job | Move FRAMES between physically adjacent nodes on ONE link |
| Frame | Layer-2 packet — encapsulates IP datagram |
| MAC Address | 48-bit, e.g. 1A-2F-BB-76-09-AD, Layer 2, same-LAN use |
| IP Address | 32-bit, Layer 3, end-to-end across Internet |
| ARP purpose | Resolve IP address → MAC address on same LAN |
| ARP Query | BROADCAST to FF-FF-FF-FF-FF-FF |
| ARP Reply | UNICAST back to requester |
| ARP Table | <IP addr, MAC addr, TTL> |
| ARP = soft state | Entry expires unless refreshed (TTL-based) |
| Routing to other LAN | IP addr stays same; MAC changes at EVERY hop |
| Frame A→R | Src MAC=A, Dst MAC=R(LAN1) |
| Frame R→B | Src MAC=R(LAN2), Dst MAC=B |
| Day in Life order | DHCP → ARP → DNS → TCP → HTTP |
| DHCP broadcast | FF:FF:FF:FF:FF:FF |
| Multiple access needed | Only for BROADCAST/shared links, not point-to-point |
| 3 MAC protocol types | Channel Partitioning, Taking Turns, Random Access |