CCN MIDTERM COMPLETE STUDY GUIDE

Computer Communications & Networks (SENS-3523 / CSNC-2413)

Weeks 1–4 | Professor-Style Notes | Roman Urdu + English

Paper Pattern: T/F | MCQ | Fill Blanks | Short (15) | Short Answers (15) | Long Numericals (30) = Total 60 Marks

# WEEK 1: Introduction to Internet, Network Architecture & Packet Capture

## 1.1 What is the Internet?

🔵 Roman Urdu: Yaar, Internet kya hai? Sochein — yeh ek 'network of networks' hai. Matlab kai saare networks milke ek bada network banate hain. Is mein har cheez — mobile, PC, server — ek dusre se baat kar sakti hai. Do tarike hain ise samajhne ke: Nuts & Bolts view aur Services view.

📘 English: The Internet is a 'network of networks' — a hierarchical structure of interconnected ISPs (Internet Service Providers). Users plug into lower-tier ISPs, which connect to upper-tier ISPs, forming the global Internet.

### Nuts & Bolts View — What makes up the Internet?

🔵 Roman Urdu: Nuts & Bolts matlab asli hardware aur software. Teen cheezein important hain: End Systems, Communication Links, aur Switching Nodes.

| Component | Description |
| --- | --- |
| End Systems (Hosts) | PC, smartphone, server — ye data generate karte ya use karte hain. Network apps yahan chalti hain. |
| Communication Links | Copper wire, fiber optic, radio, satellite — ye data carry karte hain. Har link ka apna data rate (bandwidth) hota hai. |
| Switching Nodes (Routers/Switches) | Data packets ko ek link se doosre link par forward karte hain. Internet ke routers yeh kaam karte hain. |

### Services View — Internet as a Service Provider

🔵 Roman Urdu: Services view mein Internet ek platform hai jo applications ko services deta hai — jaise ek service station. Apps hain: Web, Email, VoIP, Video Streaming, Social Media, Online Games.

📘 English: The Internet provides services to distributed applications. Apps need: reliable/unreliable delivery, minimum throughput, maximum delay, and security. The Network API (Socket API) is how applications request these services.

📝 EXAM TIP: MCQ TRAP: 'Network API' is the interface between apps and the Internet. Always-on servers have PERMANENT addresses; clients have DYNAMIC addresses.

## 1.2 What is a Protocol?

🔵 Roman Urdu: Protocol matlab pre-defined rules ka set. Jaise insaan milte hain toh 'Hi' kehte hain phir baat karte hain — network mein bhi same kaam hota hai. Protocol define karta hai: messages ka format, messages ka order, aur actions jo message milne par leni hain.

📘 English: A protocol is a pre-defined set of rules that defines: (1) Format/Syntax of messages, (2) Semantics — meaning of fields, (3) Timing/Order of messages sent and received. ALL communication in the Internet is governed by protocols.

- HTTP — Web browsing

- DNS — Domain Name Resolution

- TCP — Reliable data transfer

- UDP — Unreliable (fast) data transfer

- IP — Routing packets across Internet

- ARP — MAC address resolution

📝 EXAM TIP: IETF (Internet Engineering Task Force) defines Internet standards. RFC = Request for Comments — these are the official documents.

## 1.3 Network Structure: Edge & Core

| Part | Description |
| --- | --- |
| Network Edge | End systems (hosts, servers, apps) + Access networks (connect hosts to edge routers) |
| Network Core | Mesh of interconnected routers — routes packets from source to destination network |

### Client-Server Model vs Peer-to-Peer (P2P)

🔵 Roman Urdu: Client-Server mein: ek powerful server hamesha on hota hai (jaise web server), clients request karte hain. P2P mein: sab barabar hain, koi dedicated server nahi — BitTorrent, WhatsApp aisi applications hain.

| Model | Key Points |
| --- | --- |
| Client-Server | Server always ON, permanent IP, clients request services. Examples: Web, Email, DB |
| Peer-to-Peer (P2P) | No always-on server, peers = clients + servers. Examples: BitTorrent, Gnutella, Bitcoin |

## 1.4 Network Categorization by Coverage

🔵 Roman Urdu: Networks ko unki coverage ke hisaab se 4 categories mein divide karte hain. Yeh T/F aur MCQ mein zaroor aata hai!

| Type | Full Name & Range |
| --- | --- |
| PAN | Personal Area Network — Devices around a person (Bluetooth, ~10m) |
| LAN | Local Area Network — Building/campus (Ethernet, WiFi, up to few km) |
| MAN | Metropolitan Area Network — City-wide (DSL, Cable) |
| WAN | Wide Area Network — Country/Global (Internet) |

## 1.5 Network Architecture — Layered Approach

🔵 Roman Urdu: Network communication bohot complex hai. Ise simple karne ke liye modular approach use kiya gaya — kaam ko chhote modules mein divide karo, har module ek layer hai. Har layer neeche wali layer se services leti hai aur upar wali layer ko services deti hai.

📘 English: Layered architecture provides: (1) Modularity — change one layer without affecting others, (2) Simplified troubleshooting, (3) Allows improvement of individual modules as technology evolves.

### TCP/IP Architecture — 5 Layer Model (The Internet Standard)

| Layer | Function & Protocols |
| --- | --- |
| 5. Application | Supports network applications. Protocols: HTTP, DNS, DHCP, FTP, SMTP |
| 4. Transport | Process-to-process data transfer using port numbers. Protocols: TCP (reliable), UDP (unreliable) |
| 3. Network | Host-to-host transfer, global IP addressing, routing. Protocols: IPv4, IPv6 |
| 2. Link | Hop-to-hop transfer between neighboring devices, MAC addressing. Protocols: Ethernet, WiFi, 3G/4G |
| 1. Physical | Signal encoding, physical media, connectors — bits on wire/air |

### OSI Model (7 Layers — Older, Not Used in Practice)

🔵 Roman Urdu: OSI model 7 layers ka hai lekin practice mein TCP/IP (5 layers) use hoti hai. OSI sirf reference model hai.

📘 English: OSI = Application, Presentation, Session, Transport, Network, Data Link, Physical. TCP/IP combines Presentation+Session into Application layer.

📝 EXAM TIP: T/F: 'OSI model is used in the Internet' — FALSE. TCP/IP is used. OSI is only a reference model.

### Three Levels of Addressing in Internet

🔵 Roman Urdu: Internet mein 3 tarah ke addresses hain, har ek ek layer se related hai:

| Address Type | Layer & Description |
| --- | --- |
| Port Numbers | Transport Layer — 16-bit, identifies specific process/application (e.g., port 80 = HTTP) |
| IP Addresses | Network Layer — 32-bit (IPv4), globally unique, identifies a host |
| MAC Addresses | Link Layer — 48-bit, hardware address, identifies a NIC on local network |

### Encapsulation & Decapsulation

🔵 Roman Urdu: Jab data bheja jata hai, har layer apna header add karti hai — yeh Encapsulation hai. Jab data receive hota hai, har layer apna header remove karti hai — yeh Decapsulation hai.

📘 English: Encapsulation: Application data → Transport adds header (Segment) → Network adds header (Packet/Datagram) → Link adds header+trailer (Frame) → Physical: bits. Reverse process = Decapsulation.

📝 EXAM TIP: Terminology: Application=Message, Transport=Segment, Network=Packet/Datagram, Link=Frame

## 1.6 Packet Capturing & Analysis

🔵 Roman Urdu: Packet sniffer ek program hai jo network par aane jaane wale SARE packets pakad leta hai — chahe woh iske liye na bhi hon. NIC ko 'promiscuous mode' mein dalna hota hai. Wireshark sab se popular sniffer hai.

📘 English: Packet Sniffer: A program that captures ALL network traffic passing through the NIC, regardless of destination. Used by: (1) Network Admins — monitoring, traffic analysis, (2) Black Hat community — for attacks.

### How Packet Sniffer Works

- Runs NIC in PROMISCUOUS MODE — captures all packets (not just those addressed to it)

- Decodes packet headers and data

- Displays info for each captured packet

- Common tools: Wireshark (GUI), tcpdump (command line)

### MAC Layer Processing (Link Layer)

🔵 Roman Urdu: NIC (Network Interface Card) Ethernet frame receive karta hai. Phir Destination MAC address check karta hai — agar apna hai, upar bhejta hai. 'Ether Type' field check karta hai — IPv4 (0x0800), IPv6, ya ARP.

📘 English: NIC receives Ethernet frame → Checks Dest MAC == Own MAC → Calls Network Card Driver → Copies data to kernel memory → Checks Ether Type field → Passes to appropriate protocol handler.

| Ether Type Value | Protocol |
| --- | --- |
| 0x0800 | IPv4 |
| 0x0806 | ARP |
| 0x86DD | IPv6 |

### Network Layer Processing

🔵 Roman Urdu: IP packet handler check karta hai: Destination IP address (kya yeh mera packet hai?) aur Protocol field (upper layer protocol kya hai?). Protocol field se pata chalta hai TCP hai ya UDP.

| Protocol Field Value | Protocol |
| --- | --- |
| 6 | TCP |
| 17 | UDP |
| 1 | ICMP |

### Transport Layer Processing

🔵 Roman Urdu: TCP ya UDP handler PORT NUMBER se pata karta hai kaunsi application ko data dena hai. Port numbers: Web=80, Email=25, DNS=53, FTP=20/21.

📘 English: TCP/UDP uses Port Numbers to deliver data to the correct application. Well-known ports: HTTP=80, SMTP=25, DNS=53, FTP=21/20, HTTPS=443, Telnet=23.

# WEEK 2: Internet Application Development & Socket Programming (UDP)

## 2.1 What are Internet Applications?

🔵 Roman Urdu: Applications processes hoti hain jo end hosts par chalti hain. Agar same host par hain toh IPC (Inter Process Communication) — pipes, shared memory use hoti hai. Agar alag hosts par hain — Sockets use hote hain messages exchange karne ke liye.

📘 English: Applications are processes running on end systems. Same-host apps use Pipes/FIFOs/Shared Memory. Remote apps communicate by exchanging messages using Sockets or RPC. Examples: email, web, P2P file sharing, VoIP, gaming, video conf.

## 2.2 What is a Socket?

🔵 Roman Urdu: Socket ek software interface hai — process aur network ke beech ka darwaza. Jaise ghar ke darwaze se baahar jaate ho, process socket se network mein jaati hai. Har socket ka apna file descriptor hota hai aur kernel level pe send/receive buffers hote hain.

📘 English: Socket: Software interface between a process and the network. It is an endpoint of bidirectional communication, represented by a FILE DESCRIPTOR. Each socket has its own kernel-level send/receive buffers. Remote processes communicate by creating/reading/writing sockets.

## 2.3 Socket Address = IP Address + Port Number

🔵 Roman Urdu: Process ka address = IP address + Port number. IP address se host identify hota hai (32-bit), Port number se specific process identify hoti hai (16-bit). Yeh dono milke Socket Address banate hain.

📘 English: To receive messages over Internet, a process must have an address = IP Address (32-bit, identifies host) + Port Number (16-bit, identifies process). Socket Address = (IP Address, Port Number).

- Example: Web server at gaia.cs.umass.edu — IP: 128.119.245.12, Port: 80

📝 EXAM TIP: T/F: A socket address is just the IP address — FALSE. It is IP + Port together.

## 2.4 Port Number Ranges (VERY IMPORTANT for MCQ!)

| Range | Type & Usage |
| --- | --- |
| 0 – 1023 | Well-Known Ports — System services. HTTP:80, SMTP:25, DNS:53, HTTPS:443, FTP:21, Telnet:23, SSH:22 |
| 1024 – 49151 | Registered Ports — Vendor applications. Web Proxy:8080, MySQL:3306, Kazaa:1214 |
| 49152 – 65535 | Dynamic/Ephemeral Ports — Assigned by kernel to client processes automatically |

📘 English: Total ports: 65,536 (2^16 = 0 to 65535). Clients get ephemeral ports assigned automatically by the kernel.

📝 EXAM TIP: MCQ: Servers use Well-Known ports; Clients use Ephemeral ports. Server must know its port; client does NOT need well-known port.

## 2.5 Types of Servers

| Type | Description |
| --- | --- |
| Iterative Server | Handles ONE client at a time. Server finishes with client 1, then serves client 2. |
| Concurrent Server | Handles MULTIPLE clients simultaneously using threads or child processes (fork). |

📘 English: Both UDP and TCP can be Iterative or Concurrent. For TCP, concurrent is more common in production.

## 2.6 UDP Client-Server Communication

🔵 Roman Urdu: UDP connectionless hai — koi connection setup nahi hota. Seedha data bhejo aur seedha receive karo. sendto() aur recvfrom() use hote hain. Jab client data bhejta hai, usse server ka IP aur Port pata hona chahiye.

📘 English: UDP = connectionless, unreliable. No handshaking. Uses sendto() and recvfrom() functions. Server binds to well-known port; client creates socket and sends directly.

### UDP Server Steps

Step 1: socket() — Create UDP socket (SOCK_DGRAM)

Step 2: bind() — Bind to IP address and well-known port

Step 3: recvfrom() — Wait and receive data from any client

Step 4: Process data and sendto() reply (optional)

Step 5: close() — Close socket when done

### UDP Client Steps

Step 1: socket() — Create UDP socket (SOCK_DGRAM)

Step 2: sendto() — Send data specifying server IP + Port

Step 3: recvfrom() — Receive reply from server (optional)

Step 4: close() — Close socket

## 2.7 Socket API Functions for UDP

### socket() — Create a Socket

📐 Formula: int socket(int family, int type, int protocol)

- family: AF_INET (IPv4), AF_INET6 (IPv6)

- type: SOCK_DGRAM (UDP), SOCK_STREAM (TCP), SOCK_RAW (Raw)

- protocol: typically 0

- Returns: socket descriptor (int) on success, -1 on error

### bind() — Bind Socket to Address

📐 Formula: int bind(int sockfd, struct sockaddr* serverAddr, int addrlen)

- Binds socket to local IP address and port number

- INADDR_ANY = kernel automatically fills in host's IP

- port = 0 means kernel assigns port; else caller specifies

- Returns: 0 on success, -1 on failure

### sendto() — Send Data (UDP)

📐 Formula: int sendto(int sockfd, char* buf, size_t nbytes, int flags, struct sockaddr* to, int addrlen)

- buf: data buffer to send

- to: destination socket address (IP + Port of receiver)

- flags: set to 0 for normal behavior

- Returns: number of bytes written, -1 on failure

### recvfrom() — Receive Data (UDP)

📐 Formula: int recvfrom(int sockfd, char* buf, size_t nbytes, int flags, struct sockaddr* from, int* addrlen)

- from: filled with sender's IP and Port when function returns

- buf: buffer to store received data

- Returns: number of bytes read, -1 on failure

### close() — Close Socket

📐 Formula: int close(int sockfd)

- Marks socket as closed, no more reads/writes

- Returns: 0 on success, -1 on failure

## 2.8 Socket Address Structures

🔵 Roman Urdu: Socket API mein address structures use hote hain IP aur Port ko store karne ke liye. IPv4 ke liye sockaddr_in use hoti hai. Generic structure sockaddr_in se cast ki jaati hai.

### IPv4 Socket Address Structure (sockaddr_in)

struct sockaddr_in {     uint8_t         sin_len;      /* length of structure (16) */     sa_family_t     sin_family;   /* AF_INET */     in_port_t       sin_port;     /* 16-bit port, NETWORK BYTE ORDER */     struct in_addr  sin_addr;     /* 32-bit IPv4 addr, NETWORK BYTE ORDER */     char            sin_zero[8];  /* unused, zero it out */ }; struct in_addr {     in_addr_t s_addr;  /* 32-bit IPv4 address */ };

## 2.9 Address Conversion Functions

🔵 Roman Urdu: IP address do forms mein hoti hai: dotted decimal string (jaise '192.168.1.1') aur 32-bit binary number. In dono ke beech convert karne ke liye functions hain.

| Function | Purpose |
| --- | --- |
| inet_aton(strptr, &addr) | ASCII string to binary (dotted decimal → 32-bit). Returns 1 on success, 0 on error. |
| inet_addr(strptr) | ASCII string to 32-bit binary. Returns INADDR_NONE on error. |
| inet_ntoa(inaddr) | Binary to ASCII string (32-bit → dotted decimal). Returns pointer to string. |

## 2.10 Byte Ordering — VERY IMPORTANT!

🔵 Roman Urdu: Different CPUs alag tarike se multi-byte values store karte hain. Intel ka Little Endian hai, Network ka Big Endian hai. Jab data send karo, host byte order ko network byte order mein convert karo. Receive mein ulta karo.

📘 English: Little Endian: Low byte stored at LOW address (Intel/AMD CPUs). Big Endian: High byte stored at LOW address = Network Byte Order. Always convert before sending, convert back after receiving.

### Example of Byte Order Problem

🔵 Roman Urdu: Agar Big Endian machine 16-bit number 2 bheje aur Little Endian machine receive kare, toh woh sochegi number 512 aaya! Isliye conversion zaroori hai.

📘 English: Big Endian sends: 00000000 00000010 (= 2). Little Endian reads same bytes as: 00000010 00000000 (= 512). This is why byte order conversion functions are MANDATORY.

### Byte Ordering Functions

📐 Formula: htons(host16) — Host TO Network Short (16-bit)

📐 Formula: htonl(host32) — Host TO Network Long (32-bit)

📐 Formula: ntohs(net16) — Network TO Host Short (16-bit)

📐 Formula: ntohl(net32) — Network TO Host Long (32-bit)

📘 English: h = host, n = network, s = short (16-bit), l = long (32-bit). Header: netinet/in.h

📝 EXAM TIP: ALWAYS use htons() for port numbers and htonl() for IP addresses when filling sockaddr_in. This is a VERY common MCQ/fill-blank topic!

# WEEK 3: Access Networks, Physical Media & Network Core

## 3.1 Access Networks — Connecting Homes to Internet

🔵 Roman Urdu: Access network woh network hai jo end systems (ghar, office) ko edge router se connect karta hai. Kaafi tarah ke access networks hain — DSL, Cable, FTTH, WiFi, Cellular.

📘 English: Access networks connect end systems to the first router (edge router). They vary in: type of media used, data rate (bps), range, and whether access is shared or dedicated.

## 3.2 Home Access Networks

### (a) Digital Subscriber Line (DSL)

🔵 Roman Urdu: DSL purani telephone line use karta hai internet ke liye! Data aur voice same wire par alag frequencies par chalta hai. Splitter donon ko separate karta hai. DSLAM central office par hota hai jo data ko ISP se connect karta hai. Yeh DEDICATED line hai — share nahi hoti.

📘 English: DSL uses existing telephone line to Central Office DSLAM. Data & voice signals on same wire at different frequencies. Splitter and DSLAM separate signals. Telco = ISP. DEDICATED line (not shared).

| DSL Type | Upstream Rate |
| --- | --- |
| DSL (standard) | 128 kbps – 1 Mbps |
| VDSL (high speed) | 1.6 Mbps – 20 Mbps |

📘 English: DSL provides ASYMMETRIC access — downstream (to user) is faster than upstream (from user). Actual rate depends on link distance, wire gauge, electrical interference.

### (b) Cable Internet Access

🔵 Roman Urdu: Cable TV network use karta hai Internet ke liye. Data aur video same cable par alag frequencies par. Fiber connects cable head-end to neighborhood junctions, phir coaxial cable homes tak. SHARED medium hai — bandwidth users mein divide hoti hai.

📘 English: Uses existing Cable TV network. Cable TV provider = ISP. Fiber to neighborhood junctions, then coaxial cable to homes. Cable modem provides Ethernet connectivity.

| Feature | Cable Internet |
| --- | --- |
| Technology | HFC — Hybrid Fiber-Coax |
| Access Type | SHARED — bandwidth divided among users |
| Upstream Rate | ~2 Mbps |
| Downstream Rate | Up to 30 Mbps |
| Protocol Needed | Multiple Access Protocol (to coordinate transmission) |

📝 EXAM TIP: KEY DIFFERENCE: DSL = Dedicated line. Cable = Shared medium. This is a favorite MCQ topic!

### (c) Fiber to the Home (FTTH)

🔵 Roman Urdu: FTTH mein poora optical fiber ghar tak aata hai — bohot fast! 1 Gbps tak speed milti hai. Internet, telephone aur video teeno is par chalta hai. PON architecture mein ONT ghar mein hota hai, splitter signals combine karta hai.

📘 English: Optical fiber from home to Central Office. Rate: ~1 Gbps. Services: Internet + Phone + Video. Two architectures: (1) Direct Fiber — one fiber per home, (2) PON (Passive Optical Network) — shared fiber.

| PON Component | Function |
| --- | --- |
| ONT (Optical Network Terminator) | At home — connects home to neighborhood splitter, provides Ethernet connectivity |
| Splitter | Combines signals from multiple ONTs onto one shared fiber |
| OLT (Optical Line Terminator) | At Central Office — electro/optical conversion, connects to ISP router |

## 3.3 Enterprise Access — Ethernet (LAN)

🔵 Roman Urdu: Office ya university mein Ethernet use hoti hai. LAN (Local Area Network) end systems ko edge router se connect karta hai. Twisted pair copper wire aur switches use hote hain.

📘 English: Ethernet is the most prevalent enterprise access technology. Uses twisted-pair copper wire and LAN switches. Dedicated link from host to switch (not shared). Rates: 100 Mbps to end systems, 1-10 Gbps to servers.

## 3.4 Wireless Access Networks

| Type | Standard |
| --- | --- |
| Wireless LAN (WiFi) | 802.11b/g/n |
| Wide Area Wireless | 3G, 4G, LTE, WiMAX |

📝 EXAM TIP: WiFi = IEEE 802.11. Cellular = 3G/4G/LTE. Range and rate are opposites — higher range = lower rate (generally).

## 3.5 Physical Media

🔵 Roman Urdu: Physical media woh cheez hai jo sender aur receiver ke beech hoti hai — bits carry karta hai electrical signals, electromagnetic waves, ya optical pulses ke roop mein.

### Guided Media (Wired)

| Media Type | Details |
| --- | --- |
| Twisted Pair (TP) | Two insulated copper wires in spiral. Cat5: 100Mbps-1Gbps, Cat6: 10Gbps. Used in Ethernet LAN. |
| Coaxial Cable | Two concentric copper conductors with shielding. Bidirectional, shared medium, broadband, multiple channels. Used in Cable Internet. |
| Fiber Optic Cable | Glass fiber carrying light pulses. Rate: 10-100 Gbps. Long haul, low error, immune to electromagnetic noise. Used for backbone links. |

### Unguided Media (Wireless)

| Type | Details |
| --- | --- |
| Wireless LAN | WiFi — 802.11 — within building ~100m |
| Wide Area | 3G/4G cellular — 10s of km range |
| Satellite | Two ground stations. 100s Mbps. End-to-end delay up to 280 ms. Geostationary vs LEO (Low Earth Orbit). |

📘 English: Radio links characteristics: no physical wire, penetrate walls, support mobility. Affected by: reflection, obstruction by objects, interference.

## 3.6 The Network Core — Circuit vs Packet Switching

🔵 Roman Urdu: Network core ka fundamental sawaal: Data kaise transfer hota hai network mein? Do tarike hain: Circuit Switching (jaise telephone) aur Packet Switching (jaise Internet). Yeh topic bahut IMPORTANT hai — numericals bhi aate hain!

### Circuit Switching

🔵 Roman Urdu: Circuit switching mein pehle ek dedicated path (circuit) establish hoti hai source se destination tak. Poori transmission duration mein resources reserved rehti hain. Guaranteed service milti hai. Telephone network is ka example hai.

📘 English: Circuit switching: Before data transfer, a dedicated end-to-end circuit is established. Resources (bandwidth, buffer) reserved for entire call duration. Guaranteed performance. Idle resources are wasted (no sharing). Example: Traditional telephone network.

### Multiplexing in Circuit Switching

🔵 Roman Urdu: Circuit ek link par implement karne ke liye do tarike hain: FDM (frequency division) ya TDM (time division).

| Multiplexing | Description |
| --- | --- |
| FDM (Frequency Division Multiplexing) | Frequency spectrum divided into bands. Each call/connection gets a dedicated frequency band. Always available but narrow bandwidth. |
| TDM (Time Division Multiplexing) | Time divided into frames. Each frame has fixed slots. Each connection gets ONE slot per frame. Full bandwidth but only for that slot time. |

### NUMERICAL: TDM Data Rate Calculation

🔵 Roman Urdu: TDM circuit ki data rate calculate karna seekhte hain. Formula yaad karo!

📐 Formula: Data Rate of one TDM circuit = Frame Rate × Slot Size

📐 Formula: TDM Line Rate = Frame Rate × (Slot Size × Number of Slots)

📘 English: Example from slides: Frame rate = 8000 frames/sec, Slot size = 8 bits, 4 slots per frame

Step 1: Data rate of ONE circuit = 8000 frames/sec × 8 bits/slot = 64,000 bps = 64 kbps

Step 2: TDM Line Rate = 8000 × (8 × 4) = 8000 × 32 = 256,000 bps = 256 kbps

✅ Answer: One circuit = 64 kbps; TDM line = 256 kbps

### NUMERICAL: TDM File Transfer Time

🔵 Roman Urdu: Yeh slide ka example important hai. File transfer time ka calculation seekhte hain.

📘 English: Problem: 640 kbits file from A to B. TDM with 24 slots, line rate = 1.536 Mbps. Circuit setup time = 500 ms.

📐 Formula: Data rate of one TDM circuit = TDM Line Rate / Number of Slots

📐 Formula: File Transmission Time = File Size / Circuit Data Rate

📐 Formula: Total Time = Circuit Setup Time + File Transmission Time

Step 1: Circuit data rate = 1.536 Mbps / 24 slots = 0.064 Mbps = 64 kbps

Step 2: File Tx time = 640 kbits / 64 kbps = 10 seconds

Step 3: Total time = 500 ms + 10 sec = 10.5 seconds

✅ Answer: Total file sending time = 10.5 seconds (Note: actual time also includes propagation delay)

### PRACTICE PROBLEMS from Slides (Week 3)

### Practice Problem 1:

📘 English: A network has TDM link with 32 slots. Aggregate line rate = 2.048 Mbps. File = 1 MB. Circuit setup = 200 ms.

📐 Formula: Circuit data rate = 2.048 Mbps / 32 = 64 kbps

Step 1: Circuit data rate = 2.048 Mbps / 32 = 64 kbps

Step 2: File size = 1 MB = 8 Mbits = 8,000 kbits

Step 3: Tx time = 8,000 kbits / 64 kbps = 125 seconds

Step 4: Total = 200 ms + 125 sec = 125.2 seconds

✅ Answer: Total = 125.2 seconds

### Practice Problem 2:

📘 English: One TDM channel = 8 kbps. TDM frame has 32 slots. What is TDM line rate?

📐 Formula: TDM Line Rate = Channel Rate × Number of Slots

Step 1: TDM Line Rate = 8 kbps × 32 = 256 kbps

✅ Answer: TDM Line Rate = 256 kbps

### Practice Problem 3 (FDM):

📘 English: FDM channel data rate = 20 kbps. Line supports 12 parallel channels. App allocated 6 FDM channels.

Step 1: Data rate per app = 6 channels × 20 kbps/channel = 120 kbps

✅ Answer: Application gets 120 kbps

### Packet Switching

🔵 Roman Urdu: Packet switching mein koi dedicated circuit nahi hoti. Data ko chhote packets mein toda jata hai. Har packet independently route hota hai. Resources on-demand use hote hain — Statistical Multiplexing kehte hain ise. Internet yahi use karta hai.

📘 English: Packet switching: Application messages broken into PACKETS. Each packet uses FULL link data rate. Network resources used ON DEMAND. Routers use Store-and-Forward: entire packet must arrive before forwarding.

| Feature | Circuit Switching |
| --- | --- |
| Resource Allocation | DEDICATED (reserved) |
| Guarantee | Guaranteed bandwidth |
| Idle Resource | Wasted |
| Example | Telephone PSTN |
| Setup Required | YES (call setup) |
| Used For | Voice calls (constant rate) |

### Statistical Multiplexing

🔵 Roman Urdu: Packet switching mein statistical multiplexing hoti hai — resources on-demand allocate hote hain based on statistical pattern of arrivals. TDM mein pre-allocated slots the, yahan nahi.

📘 English: Statistical Multiplexing: Packets of A and B do NOT have fixed time slots. Based on demand, any packet can use any slot at any time. More efficient than TDM for bursty traffic.

### Packet vs Circuit: User Capacity Example

📘 English: 10 users, 1 Mbps link. Each user needs 100 kbps when active, but active only 10% of time.

Step 1: Circuit switching: 1 Mbps / 100 kbps = can support only 10 users (each gets fixed 100 kbps)

Step 2: Packet switching: With 10% activity, on average only 1 user active at a time. Can potentially support 35+ users with <0.0004 probability of congestion

✅ Answer: Packet switching allows MORE users for bursty traffic (great for internet data)!

📝 EXAM TIP: Packet switching disadvantage: congestion possible — packets may be delayed or lost. Needs protocols for reliability.

## 3.7 Key Network Core Functions: Forwarding vs Routing

| Function | Description |
| --- | --- |
| Forwarding | LOCAL action — move packets from router's INPUT port to appropriate OUTPUT port using forwarding table. |
| Routing | GLOBAL action — determine source-to-destination route using routing algorithms (e.g., LS, DV). |

📝 EXAM TIP: Forwarding = data plane (fast, hardware). Routing = control plane (slow, software). Very commonly asked in MCQ.

# WEEK 4 (Part 1): Packet Delays & Throughput

## 4.1 Four Sources of Packet Delay

🔵 Roman Urdu: Jab packet ek router se dusre router tak travel karta hai, usse 4 tarah ki delays face karni padti hain. Yeh BOHOT important hai — har exam mein aata hai!

📐 Formula: Total Delay = d_proc + d_queue + d_trans + d_prop

| Delay Type | Symbol |
| --- | --- |
| Processing Delay | d_proc |
| Queuing Delay | d_queue |
| Transmission Delay | d_trans |
| Propagation Delay | d_prop |

### Transmission Delay — Detailed

🔵 Roman Urdu: Transmission delay woh time hai jo first bit wire par daalne se last bit daalne tak lagta hai. Yeh LINK KI SPEED par depend karta hai — actual distance se kuch lena dena nahi!

📐 Formula: d_trans = L / R

- L = Packet size in BITS

- R = Link data rate in bits per second (bps)

- Example: L = 1 KB = 8000 bits, R = 500 kbps → d_trans = 8000/500,000 = 0.016 s = 16 ms

📝 EXAM TIP: Tx delay depends ONLY on packet size and link rate. NOT on distance!

### Propagation Delay — Detailed

🔵 Roman Urdu: Propagation delay woh time hai jo bit ko wire ya air mein travel karne mein lagta hai. Yeh DISTANCE par depend karta hai. Speed ~3×10^8 m/s hai (light speed).

📐 Formula: d_prop = d / s

- d = Physical length of link in meters

- s = Propagation speed ≈ 3×10^8 m/s (in fiber: ~2×10^8 m/s)

- Example: d = 200 km = 200,000 m, s = 2.5×10^8 m/s → d_prop = 200,000/250,000,000 = 0.8 ms

📝 EXAM TIP: Propagation delay depends ONLY on distance. NOT on packet size or link rate!

### Queuing Delay

🔵 Roman Urdu: Queuing delay tab hoti hai jab packets queue mein wait karte hain output link par transmission ka. Agar router buffer bhar jaye toh packets drop ho jaate hain — yeh PACKET LOSS hai! Lost packet may need retransmission.

📘 English: If packet arrival rate at input > transmission rate of output link → packets queue up. If buffer full → packet dropped (PACKET LOSS). Lost packet may be retransmitted by previous node, source, or not at all.

### Store-and-Forward in Packet Switching

🔵 Roman Urdu: Router packet ko tab tak forward nahi karta jab tak poora packet receive na ho jaye — yeh Store-and-Forward principle hai.

📘 English: Store-and-Forward: Router must receive ENTIRE packet before forwarding to next link. This adds 1×(L/R) delay at each router for each packet.

## 4.2 Traceroute

🔵 Roman Urdu: Traceroute ek program hai jo source se destination tak har router par delay measure karta hai. Har router ko 3 probe packets bhejta hai — delays measure karta hai. '*' ka matlab hai router ne reply nahi kiya.

📘 English: Traceroute: Sends 3 packets to each router along the path to destination. Each router returns packets to sender. Sender times interval between send and reply. Output shows delay to each hop. '*' = no response (router not replying to traceroute).

## 4.3 Throughput

🔵 Roman Urdu: Throughput matlab actual mein kitna data per second transfer ho raha hai. Instantaneous thruput kisi bhi waqt ka rate hai, Average thruput total data divided by total time.

📐 Formula: Average Throughput (bps) = File Size (bits) / Delay (seconds)

### Bottleneck Link

🔵 Roman Urdu: End-to-end path mein jo link sab se slow hai, woh BOTTLENECK link hai. Poora end-to-end throughput bottleneck link ki speed se limit hota hai — chahe baaki links kitni bhi fast kyun na hon!

📘 English: Bottleneck link: The slowest link on end-to-end path that limits average throughput. If path has links R1, R2, R3: Throughput = min(R1, R2, R3).

📐 Formula: End-to-end Throughput = min(R_c, R_s)  [for simple two-link case]

📐 Formula: With shared backbone R: Throughput = min(R_c, R_s, R/10)  [for 10 connections]

📝 EXAM TIP: In practice: The access link (Rc or Rs) is usually the bottleneck, not the backbone.

## 4.4 FULL PRACTICE PROBLEM (From Slides — Week 4)

📘 English: Host A → Router 1 → Router 2 → Host B. Links: R1=500 kbps, R2=2 Mbps, R3=4 Mbps. Link lengths: d1=100m, d2=200km, d3=10km. Propagation speed s=2.5×10^8 m/s. Queuing delay=50ms each router, Proc delay=20ms each router.

### Part (a): Average Throughput

📐 Formula: Bottleneck = min(R1, R2, R3) = min(500 kbps, 2 Mbps, 4 Mbps) = 500 kbps = R1

✅ Answer: Average Throughput = 500 kbps (limited by R1, the bottleneck link)

### Part (b): Delay for 1 Packet (L=1 KB=8000 bits)

📘 English: Calculate ALL transmission delays:

Step 1: d_trans(Link1) = L/R1 = 8000 bits / 500,000 bps = 0.016 s = 16 ms

Step 2: d_trans(Link2) = L/R2 = 8000 bits / 2,000,000 bps = 0.004 s = 4 ms

Step 3: d_trans(Link3) = L/R3 = 8000 bits / 4,000,000 bps = 0.002 s = 2 ms

Step 4: Total d_trans = 16 + 4 + 2 = 22 ms

📘 English: Calculate ALL propagation delays:

Step 5: d_prop(Link1) = d1/s = 100 m / 2.5×10^8 = 0.0000004 s ≈ 0.0004 ms

Step 6: d_prop(Link2) = d2/s = 200,000 m / 2.5×10^8 = 0.0008 s = 0.8 ms

Step 7: d_prop(Link3) = d3/s = 10,000 m / 2.5×10^8 = 0.00004 s = 0.04 ms

Step 8: Total d_prop = 0.0004 + 0.8 + 0.04 ≈ 0.8404 ms

Step 9: Total 1-pkt delay = d_trans + d_prop = 22 + 0.8404 = 22.8404 ms (no queuing/proc for 1 pkt alone)

✅ Answer: Single packet end-to-end delay ≈ 22.84 ms

### Part (c): File Transfer Time (File = 4 MB, pkt = 1 KB, with delays)

Step 1: Number of packets = 4 MB / 1 KB = 4,000,000 / 1,000 = 4000 packets

Step 2: Tx delay for 3999 packets at Host A (Link 1) = 3999 × L/R1 = 3999 × 16 ms = 63,984 ms

Step 3: Last packet delay (A to B) = 22.8404 ms (from part b)

Step 4: Queuing delay at 2 routers = 2 × 50 ms = 100 ms

Step 5: Processing delay at 2 routers = 2 × 20 ms = 40 ms

Step 6: Total = 63,984 + 22.8404 + 100 + 40 = 64,146.84 ms ≈ 64.15 seconds

✅ Answer: Total file transfer time ≈ 64.15 seconds

### Part (d): Achieved Throughput

📐 Formula: Throughput = File Size / Total Time = (4 × 10^6 × 8 bits) / 64.15 sec

Step 1: File size in bits = 4,000,000 bytes × 8 = 32,000,000 bits = 32 Mbits

Step 2: Throughput = 32,000,000 / 64.15 ≈ 498,800 bps ≈ 498.8 kbps

✅ Answer: Achieved throughput ≈ 498.8 kbps (close to bottleneck rate of 500 kbps, as expected)

# WEEK 4 (Part 2): Application Layer Principles & TCP Socket Programming

## 4.5 Services Required by Applications

🔵 Roman Urdu: Mukhtalif applications ko network se mukhtalif services chahiye hoti hain. Kuch reliable data transfer chahte hain, kuch real-time delivery chahte hain, kuch high throughput chahte hain.

| Service Requirement | Description & App Types |
| --- | --- |
| Reliability | Loss-intolerant apps: Email, File transfer, Web (need complete, error-free delivery). Loss-tolerant apps: Audio/Video streaming (can tolerate some loss). |
| Throughput | Bandwidth-intensive apps: VoIP, Video conferencing (need minimum throughput). Elastic apps: Web, Email (use whatever is available). |
| Delay | Real-time interactive apps: VoIP, teleconference, games (tight delay constraints <150ms). Non-real-time: bulk email, file transfer (tolerate delays). |
| Security | Confidentiality (encryption), Data integrity (no tampering), Source authentication. |

## 4.6 TCP vs UDP Services — CRITICAL COMPARISON!

🔵 Roman Urdu: Yeh comparison bohot important hai. TCP reliable hai lekin slow — connection setup lagti hai. UDP unreliable hai lekin fast — koi setup nahi. Har application apne need ke hisaab se choose karti hai.

| Feature | TCP |
| --- | --- |
| Reliability | YES — reliable transport, no loss |
| Connection | Connection-oriented: setup REQUIRED |
| Flow Control | YES — sender won't overwhelm receiver |
| Congestion Control | YES — won't overload network |
| Delay Guarantee | NO |
| Throughput Guarantee | NO |
| Security | NO (need TLS/SSL) |
| Speed | Slower (due to overhead) |
| Example Apps | HTTP, FTP, SMTP, Email |

📝 EXAM TIP: MCQ: Why use UDP if it's unreliable? Because: (1) Some apps prefer SPEED over reliability (VoIP, gaming), (2) Apps implement their own reliability, (3) Less overhead = lower latency

## 4.7 App Layer Protocols vs Applications

🔵 Roman Urdu: Application aur App-layer protocol alag cheezein hain. Web browser ek application hai. HTTP ek app-layer protocol hai jo browser use karta hai. Protocol define karta hai message types, syntax, semantics, aur timing.

| App-layer Protocol Defines | Description |
| --- | --- |
| Types of Messages | Request or Response |
| Message Syntax | Fields in messages and how they are delineated |
| Message Semantics | Meaning of information in the fields |
| Timing/Rules | When and how processes send/receive messages |
| Protocol Type | Description & Examples |
| Open Protocols | Defined in RFCs, allow interoperability. Examples: HTTP (RFC 2616), FTP (RFC 959) |
| Proprietary Protocols | Not publicly defined. Examples: Skype, MS Exchange Server protocols |

## 4.8 Application Architectures

| Architecture | Client-Server |
| --- | --- |
| Server | Always-on, permanent IP, data centers for scaling |
| Client | Intermittent, dynamic IP, requests service |
| Scalability | Requires infrastructure investment |
| Management | Easier to manage |
| Examples | Web (HTTP), Email, DNS |
| Challenge | Infrastructure cost, potential bottleneck |

## 4.9 TCP Socket Programming — Connection Oriented

🔵 Roman Urdu: TCP mein pehle connection establish hoti hai phir data bheja jata hai. Teen-way handshake hoti hai: SYN → SYN-ACK → ACK. Connection band karne mein 4-way exchange hoti hai: FIN → ACK → FIN → ACK.

### TCP Connection Establishment (3-Way Handshake)

Step 1: Client calls connect() → sends SYN segment to server (Active Open)

Step 2: Server (blocked in accept()) receives SYN → sends SYN+ACK back

Step 3: Client receives SYN+ACK → connect() returns → sends ACK

Step 4: Server's accept() returns with new connected socket descriptor

### TCP Connection Termination (4-Way)

Step 1: Client calls close() → sends FIN (Active Close)

Step 2: Server receives FIN → sends ACK. Server's read() returns 0.

Step 3: Server calls close() → sends FIN (Passive Close)

Step 4: Client receives FIN → sends ACK. Connection fully terminated.

### TCP Connection Socket Pair

🔵 Roman Urdu: TCP connection ek unique socket pair se identify hoti hai: (Client IP, Client Port, Server IP, Server Port).

📘 English: TCP connection defined by: Client socket address (IP:Port) + Server socket address (IP:Port). Example: (128.2.194.242:3479, 208.216.181.15:80). This 4-tuple UNIQUELY identifies every TCP connection.

## 4.10 TCP Server Steps

Step 1: socket() — Create TCP socket (SOCK_STREAM)

Step 2: bind() — Bind to IP address and well-known port

Step 3: listen() — Put socket in listening state (passive open), set backlog queue

Step 4: accept() — Wait for client connection; returns new connected socket descriptor

Step 5: send()/recv() or write()/read() — Exchange data with client

Step 6: close() — Close new_fd (connected socket); loop back to accept() for next client

## 4.11 TCP Client Steps

Step 1: socket() — Create TCP socket (SOCK_STREAM)

Step 2: connect() — Send connection request to server (SYN sent automatically)

Step 3: send()/recv() or write()/read() — Exchange data

Step 4: close() — Close socket (FIN sent automatically)

## 4.12 TCP-Specific Socket API Functions

### listen() — Wait for Connections (Server Only)

📐 Formula: int listen(int sockfd, int backlog)

- Puts server socket into PASSIVE OPEN / listening state

- backlog = max number of pending (un-accept()ed) connections in queue

- Returns: 0 on success, -1 on failure

### connect() — Send Connection Request (Client Only)

📐 Formula: int connect(int sockfd, struct sockaddr* servaddr, int addrlen)

- Client initiates TCP 3-way handshake (sends SYN)

- servaddr: IP and port of server

- Returns: 0 on success (when connection established), -1 on failure

### accept() — Accept Connection (Server Only)

📐 Formula: int accept(int sockfd, struct sockaddr* cliaddr, int* addrlen)

- sockfd = LISTENING socket descriptor

- Returns: NEW connected socket descriptor (different from listening socket!)

- cliaddr: filled with client's IP and port number when accept returns

- addrlen: value-result argument — caller passes structure size, kernel fills bytes stored

📝 EXAM TIP: KEY: accept() returns TWO sockets conceptually: (1) Listening socket (sockfd) remains for future clients. (2) NEW connected socket (new_fd) for THIS client only.

### send() / write() — Send Data (TCP)

📐 Formula: int write(int sockfd, char* buf, size_t nbytes)

📐 Formula: int send(int sockfd, char* buf, size_t nbytes, int flags)

- Write data to STREAM socket (TCP)

- Returns number of bytes written, or -1 on failure

### recv() / read() — Receive Data (TCP)

📐 Formula: int read(int sockfd, char* buf, size_t nbytes)

📐 Formula: int recv(int sockfd, char* buf, size_t nbytes, int flags)

- Read data from STREAM socket (TCP)

- Returns: bytes read, -1 on error, 0 if OTHER END closed socket

# QUICK REVISION: T/F, MCQ & Fill in the Blanks

🔵 Roman Urdu: Ab exam pattern ke hisaab se quick revision karte hain. Yeh sab common T/F, MCQ, aur fill blank questions hain jo aate hain!

## True / False Practice

| Statement | Answer |
| --- | --- |
| The Internet is a single network. | FALSE — It is a 'network of networks' |
| OSI model is used in the Internet. | FALSE — TCP/IP (5-layer) is used; OSI is only reference |
| DSL uses SHARED medium for Internet access. | FALSE — DSL is DEDICATED (Cable is shared) |
| Propagation delay depends on packet size. | FALSE — Propagation delay = d/s, depends on DISTANCE only |
| Transmission delay depends on link distance. | FALSE — Tx delay = L/R, depends on PACKET SIZE and LINK RATE |
| TCP provides delay and throughput guarantees. | FALSE — TCP provides reliability, flow/congestion control, but NOT delay/throughput guarantee |
| UDP is connection-oriented. | FALSE — UDP is connectionless |
| Port 80 is used for HTTP. | TRUE |
| Well-known ports range from 1024-49151. | FALSE — Well-known ports: 0-1023; 1024-49151 are Registered ports |
| A socket address consists of IP address only. | FALSE — Socket address = IP + Port number |
| Packet switching provides guaranteed bandwidth. | FALSE — Packet switching is best-effort; Circuit switching provides guarantees |
| Routers operate at the Network layer. | TRUE |
| In Store-and-Forward, router forwards packets bit by bit. | FALSE — Router waits for ENTIRE packet before forwarding |
| Fiber optic is immune to electromagnetic noise. | TRUE |
| htons() converts 32-bit values. | FALSE — htons() converts 16-bit (short). htonl() converts 32-bit. |

## Fill in the Blanks Practice

| Blank | Answer |
| --- | --- |
| Total port numbers available = ___ | 65,536 (2^16) |
| Well-known ports range: ___ to ___ | 0 to 1023 |
| Ephemeral/Dynamic ports range: ___ to ___ | 49152 to 65535 |
| TCP uses socket type ___; UDP uses ___ | SOCK_STREAM; SOCK_DGRAM |
| Formula for Transmission Delay = ___ | L/R (L=packet size in bits, R=link rate in bps) |
| Formula for Propagation Delay = ___ | d/s (d=distance, s=propagation speed) |
| Total Packet Delay = ___ | d_proc + d_queue + d_trans + d_prop |
| Internet standards are published as ___ by ___ | RFCs; IETF |
| Network Byte Order is same as ___ Endian | Big |
| HFC stands for ___ | Hybrid Fiber-Coax (used in Cable Internet) |
| PON stands for ___ | Passive Optical Network (used in FTTH) |
| FDM = ___ ; TDM = ___ | Frequency Division Multiplexing; Time Division Multiplexing |
| Forwarding = ___-plane; Routing = ___-plane | Data; Control |
| HTTP port = ___, DNS port = ___, FTP port = ___ | 80, 53, 21 |
| The bottleneck link is the ___ link on the path | slowest |
| INADDR_ANY means ___ | kernel automatically fills in the host's own IP address |
| listen() puts socket in ___ state | passive open / listening |
| accept() returns a ___ socket descriptor | new connected (different from listening socket) |
| Ether Type 0x0800 means ___ | IPv4 |
| Protocol field value 6 = ___, value 17 = ___ | TCP; UDP |

## Common MCQ Topics Summary

| Topic | Key Point to Remember |
| --- | --- |
| TCP vs UDP | TCP=reliable+connection; UDP=unreliable+connectionless+faster |
| Circuit vs Packet | Circuit=dedicated+guaranteed; Packet=shared+best-effort+efficient |
| DSL vs Cable | DSL=dedicated; Cable=shared HFC |
| Layering | 5 layers TCP/IP: App→Transport→Network→Link→Physical |
| Addressing | Port(Transport), IP(Network), MAC(Link) |
| Byte Order | htons for ports(16-bit), htonl for IP(32-bit); Network=Big Endian |
| Socket Steps UDP | socket→bind(server)→sendto/recvfrom→close |
| Socket Steps TCP Server | socket→bind→listen→accept→send/recv→close |
| Socket Steps TCP Client | socket→connect→send/recv→close |
| Throughput | = min(all link rates) = bottleneck link rate |

# NUMERICAL PROBLEMS BANK — For Long Questions (30 Marks)

🔵 Roman Urdu: Yeh sare numericals exam ke long questions ke liye important hain. Har numerical mein formula pehle, phir step-by-step solution likha hai.

## Numerical 1: Transmission Delay Only

📘 English: A 1 MB file is sent over a 1 Mbps link. What is the transmission delay?

📐 Formula: d_trans = L / R

Step 1: Convert file: 1 MB = 1 × 10^6 × 8 = 8 × 10^6 bits = 8 Mbits

Step 2: R = 1 Mbps = 1 × 10^6 bps

Step 3: d_trans = 8 × 10^6 bits / 1 × 10^6 bps = 8 seconds

✅ Answer: Transmission Delay = 8 seconds

## Numerical 2: Propagation Delay Only

📘 English: A link is 5000 km long. Propagation speed = 2×10^8 m/s. Find propagation delay.

📐 Formula: d_prop = d / s

Step 1: d = 5000 km = 5000 × 1000 = 5,000,000 m = 5 × 10^6 m

Step 2: s = 2 × 10^8 m/s

Step 3: d_prop = 5 × 10^6 / 2 × 10^8 = 0.025 s = 25 ms

✅ Answer: Propagation Delay = 25 ms

## Numerical 3: Total Delay (Single Link)

📘 English: Packet of 1500 bytes, link rate 10 Mbps, link length 1000 km, prop speed 2×10^8 m/s, processing=1ms, queuing=5ms.

📐 Formula: Total Delay = d_proc + d_queue + d_trans + d_prop

Step 1: L = 1500 bytes = 12,000 bits; R = 10 Mbps = 10^7 bps

Step 2: d_trans = 12,000 / 10^7 = 0.0012 s = 1.2 ms

Step 3: d = 1000 km = 10^6 m; d_prop = 10^6 / 2×10^8 = 0.005 s = 5 ms

Step 4: d_proc = 1 ms; d_queue = 5 ms

Step 5: Total = 1 + 5 + 1.2 + 5 = 12.2 ms

✅ Answer: Total Packet Delay = 12.2 ms

## Numerical 4: TDM Circuit Data Rate

📘 English: TDM system with 30 slots, frame rate = 8000 frames/sec, each slot = 8 bits.

📐 Formula: Single circuit data rate = Frame Rate × Slot Size

📐 Formula: TDM Line Rate = Frame Rate × (Slot Size × Number of Slots)

Step 1: Single circuit = 8000 × 8 = 64,000 bps = 64 kbps

Step 2: TDM Line = 8000 × (8 × 30) = 8000 × 240 = 1,920,000 bps = 1.92 Mbps

✅ Answer: Single circuit = 64 kbps; TDM Line Rate = 1.92 Mbps

## Numerical 5: Throughput Calculation

📘 English: A file of 500 MB is transferred between two hosts. Transfer takes 200 seconds. Find average throughput.

📐 Formula: Throughput = File Size (bits) / Time (seconds)

Step 1: File = 500 MB = 500 × 10^6 × 8 bits = 4 × 10^9 bits = 4 Gbits

Step 2: Throughput = 4 × 10^9 / 200 = 2 × 10^7 bps = 20 Mbps

✅ Answer: Average Throughput = 20 Mbps

## Numerical 6: Bottleneck Throughput

📘 English: Host A → Host B through 4 links with rates 100 Mbps, 1 Gbps, 500 Mbps, 10 Mbps. Find end-to-end throughput.

📐 Formula: Throughput = min(all link rates)

Step 1: min(100 Mbps, 1000 Mbps, 500 Mbps, 10 Mbps) = 10 Mbps

✅ Answer: End-to-end Throughput = 10 Mbps (bottleneck = last link at 10 Mbps)

## Numerical 7: Packet Switching Users vs Circuit Switching

📘 English: 100 users share a 10 Mbps link. Each user is active 20% of time and needs 1 Mbps when active. (a) How many users can circuit switching support? (b) With packet switching, what is probability all 10 simultaneously active users exceed circuit limit?

Step 1: (a) Circuit: 10 Mbps / 1 Mbps = 10 users max (each gets fixed 1 Mbps slice)

Step 2: (b) Packet: With 100 users, each active 20%, expected active = 100 × 0.20 = 20 users

Step 3: Packet switching can serve all users as long as ≤10 are active simultaneously

✅ Answer: Circuit = 10 users; Packet switching handles 100 users (with occasional congestion if >10 active at once)

# FINAL EXAM TIPS — Professor's Advice

🔵 Roman Urdu: Exam se pehle yeh cheezein dobara zaroor dekhna — yeh sab important hain!

| Must Know | Details |
| --- | --- |
| All 4 delay formulas | d_proc + d_queue + d_trans(L/R) + d_prop(d/s) |
| TDM numerical steps | Circuit rate = Line rate / slots. File time = size / circuit rate + setup |
| Throughput = bottleneck | Always min of all link rates on path |
| TCP vs UDP table | All 7 differences — reliability, connection, flow/congestion control |
| Socket steps UDP vs TCP | UDP: socket→bind→sendto/recvfrom. TCP: socket→bind→listen→accept→read/write |
| Port ranges | 0-1023 well-known, 1024-49151 registered, 49152-65535 ephemeral |
| TCP/IP 5 layers | App→Transport→Network→Link→Physical with protocols for each |
| Circuit vs Packet | Dedicated vs on-demand, guaranteed vs best-effort, telephone vs internet |
| Byte order functions | htons/htonl (send), ntohs/ntohl (receive) |
| IETF & RFC | Internet standards body and standards documents |

Best of Luck for your Midterm Exam! You've got this! 🎯

Prepared based on: Week 1-4 slides | CCN SENS-3523/CSNC-2413 | Dr. Nauman Mazhar, UCP